Pixel barrel

The pixel barrel consists of:

- 3 barrels

- Services

The coordinates of barrels are in next table

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| B-layer | -400.00 | 400.00 | 45.50 | 74.00 |
| Layer 1 | -400.00 | 400.00 | 83.00 | 111.00 |
| Layer 2 | -400.00 | 400.00 | 117.00 | 144.00 |

I use a three dimensional coordinate system (X, Y, Z) for module parts.

The coordinate system is following:

- Axis X – perpendicular to Z-axis, direction – up

- Axis Y – perpendicular to X- and Z-axis, direction - forward

- Axis Z – parallel to beam pipe, direction – right

The shape of module parts is box.

DX, DY, DZ is half of box dimensions.

The each layer contains ladders. The dimensions of ladders are in next table.

The number of ladders for individual layers is:

- 22 - for b-layer

- 38 - for 1st  layer

- 52 - for 2nd layer

|  | DX[mm] | DY[mm] | DZ[mm] | Angle[Deg] |
| --- | --- | --- | --- | --- |
| Ladder of b-layer | 1.18 | 9.30 | 400.00 | -20.00 |
| Ladder of layer 1 | 1.21 | 9.30 | 400.00 | -20.00 |
| Ladder of layer 2 | 1.21 | 9.30 | 400.00 | -20.00 |

The position of 1st ladder in individual layers (the highest one) is in following table. There are three rows, 1st one for b-layer, 2nd one for 1-layer and 3rd one for 2-layer.

All tables with rows "The centre coordinates" show the center coordinates, where are situated the appropriate parts.

|  | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| The centre coordinates | 50.50 | 0.00 | 0.00 |
| The centre coordinates | 88.50 | 0.00 | 0.00 |
| The centre coordinates | 122.50 | 0.00 | 0.00 |

There are boxes of cables over the ladder of modules where are situated cables for modules. The dimensions and coordinates of them are in next 2 tables

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Box of cables | 0.70 | 7.00 | 400.00 |
| Box of cables | 0.70 | 7.00 | 400.00 |
| Box of cables | 0.70 | 7.00 | 400.00 |

|  | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| The centre coordinates | 52.50 | 0.00 | 0.00 |
| The centre coordinates | 90.50 | 0.00 | 0.00 |
| The centre coordinates | 124.50 | 0.00 | 0.00 |

The size of one module is in next table. The modules in b-layer differ from modules in layer 1,2.

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Module (b-layer) | 0.35 | 9.30 | 31.50 |
| Module (layer 1) | 0.38 | 9.30 | 31.50 |
| Module (layer 2) | 0.38 | 9.30 | 31.50 |

There are situated 13 modules along Z-axis in layers. The 6 modules are rotated under degree -1.10, the 7th module isn't rotated and the rest 6 modules are rotated under degree 1.10. There are center coordinates for b-layer in next tables. For other layers the Z- and Y-coordinates are same, the X-coordinates have to be replaced. (X=88.50 for layer1, X=122.50 for layer 2)

| The center coordinates | X[mm] | Y[mm] | Z[mm] | Angle[Deg] |
| --- | --- | --- | --- | --- |
| Module 1 | 50.50 | 0.00 | 370.20 | -1.10 |
| Module 2 | 50.50 | 0.00 | 308.50 | -1.10 |
| Module 3 | 50.50 | 0.00 | 246.80 | -1.10 |
| Module 4 | 50.50 | 0.00 | 185.10 | -1.10 |
| Module 5 | 50.50 | 0.00 | 123.40 | -1.10 |
| Module 6 | 50.50 | 0.00 | 61.70 | -1.10 |
| Module 7 | 50.50 | 0.00 | 0.00 | 0.00 |
| Module 8 | 50.50 | 0.00 | -61.70 | 1.10 |
| Module 9 | 50.50 | 0.00 | -123.40 | 1.10 |
| Module 10 | 50.50 | 0.00 | -185.10 | 1.10 |
| Module 11 | 50.50 | 0.00 | -246.80 | 1.10 |
| Module 12 | 50.50 | 0.00 | -308.50 | 1.10 |
| Module 13 | 50.50 | 0.00 | -370.20 | 1.10 |

There are ladders that hold pixel module in next table. The ladders are same for all layers.

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Pixel barrel ladder | 0.46 | 7.00 | 400.00 |
| Pixel barrel ladder | 0.46 | 7.00 | 400.00 |
| Pixel barrel ladder | 0.46 | 7.00 | 400.00 |

|  | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| The centre coordinates | 52.25 | 0.00 | 0.00 |
| The centre coordinates | 90.25 | 0.00 | 0.00 |
| The centre coordinates | 124.25 | 0.00 | 0.00 |

The cables for individual modules are presented in next tables. There are dimensions of cables for individual modules.

| Cables | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Module 1 | 0.015 | 2.50 | 14.90 |
| Module 2 | 0.015 | 2.50 | 45.75 |
| Module 3 | 0.015 | 2.50 | 76.60 |
| Module 4 | 0.015 | 2.50 | 107.45 |
| Module 5 | 0.015 | 2.50 | 138.30 |
| Module 6 | 0.015 | 2.50 | 169.15 |
| Module 7 | 0.015 | 2.50 | 200.00 |
| Module 8 | 0.015 | 2.50 | 169.15 |
| Module 9 | 0.015 | 2.50 | 138.30 |
| Module 10 | 0.015 | 2.50 | 107.45 |
| Module 11 | 0.015 | 2.50 | 76.60 |
| Module 12 | 0.015 | 2.50 | 45.75 |
| Module 13 | 0.015 | 2.50 | 14.90 |

There is center position of cables for b-layer in following table. (Y- and Z- coordinates are same for all layers, the X-coordinates have to be replaced: layer 1, module 1  - instead 52.75 use 90.75;

layer 1,  module 2 - instead 52.79 use 90.79,etc;

layer 2,  module 1 - instead 52.75 use 124.75, etc.)

| Cables | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| Module 1 | 52.75 | 0.00 | 385.10 |
| Module 2 | 52.79 | 0.00 | 354.25 |
| Module 3 | 52.82 | 0.00 | 323.40 |
| Module 4 | 52.85 | 0.00 | 292.55 |
| Module 5 | 52.88 | 0.00 | 261.70 |
| Module 6 | 52.91 | 0.00 | 230.85 |
| Module 7 | 52.94 | 0.00 | 200.00 |
| Module 8 | 52.98 | 0.00 | -230.85 |
| Module 9 | 53.01 | 0.00 | -261.70 |
| Module 10 | 53.04 | 0.00 | -292.55 |
| Module 11 | 53.07 | 0.00 | -323.40 |
| Module 12 | 53.10 | 0.00 | -354.25 |
| Module 13 | 53.14 | 0.00 | -385.10 |

The pixel module consists of:

- Hybrid with electronics

- Silicon crystal

- FE electronics

The dimensions and positions of these parts are written in following table

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Pixel hybrid with electronics | 0.12 | 7.00 | 30.70 |
| Pixel hybrid with electronics | 0.12 | 7.00 | 30.70 |
| Pixel hybrid with electronics | 0.12 | 7.00 | 30.70 |

The X- and Y- coordinates are valid for all modules on layers. The Z-coordinates are valid only for 7th module. The other Z-coordinates have to be taken from table of modules.

|  | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| The centre coordinates | 50.25 | 0.00 | 0.00 |
| The centre coordinates | 88.25 | 0.00 | 0.00 |
| The centre coordinates | 122.25 | 0.00 | 0.00 |

The pixel silicon crystal (the coordinates are in 1st table) consists of inactive and active part. The outer part is inactive, inner part is active (the coordinates of active part are in 2nd table). The center position for these parts is in 3rd table.

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Pixel silicon crystal | 0.10 | 9.30 | 31.50 |
| Pixel silicon crystal | 0.13 | 9.30 | 31.50 |
| Pixel silicon crystal | 0.13 | 9.30 | 31.50 |

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Active area of silicon | 0.10 | 8.20 | 30.40 |
| Active area of silicon | 0.13 | 8.20 | 30.40 |
| Active area of silicon | 0.13 | 8.20 | 30.40 |

The X- and Y- coordinates are valid for all modules on layers. The Z-coordinates are valid only for 7th module. The other Z-coordinates have to be taken from table of modules.

|  | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| The centre coordinates | 50.50 | 0.00 | 0.00 |
| The centre coordinates | 88.50 | 0.00 | 0.00 |
| The centre coordinates | 122.50 | 0.00 | 0.00 |

The dimensions and coordinates of electronics are in next two tables

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Pixel FE electronics | 0.06 | 7.00 | 30.30 |
| Pixel FE electronics | 0.00 | 7.00 | 30.30 |
| Pixel FE electronics | 0.00 | 7.00 | 30.30 |

The X- and Y- coordinates are valid for all modules on layers. The Z-coordinates are valid only for 7th module. The other Z-coordinates have to be taken from table of modules.

|  | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| The centre coordinates | 50.68 | 0.00 | 0.00 |
| The centre coordinates | 88.70 | 0.00 | 0.00 |
| The centre coordinates | 122.70 | 0.00 | 0.00 |

Services

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] | Multiplicity | Material |
| --- | --- | --- | --- | --- | --- | --- |
| Services 1 | -440.00 | 440.00 | 68.00 | 68.60 | 1 | Carbon |
| Services 2 | 400.00 | 440.00 | 50.50 | 51.40 | 2 | Mat_srv |
| Services 3 | 440.00 | 442.00 | 50.50 | 70.00 | 2 | Mat_srv |
| Services 4 | 440.00 | 450.00 | 70.00 | 71.70 | 2 | Mat_srv |
| Services 5 | 420.01 | 420.59 | 52.50 | 88.50 | 2 | Carbon |
| Services 6 | 443.00 | 450.00 | 58.00 | 58.80 | 2 | Carbon |
| Services 7 | -440.00 | 440.00 | 95.70 | 96.30 | 1 | Carbon |
| Services 8 | 400.00 | 440.00 | 88.50 | 89.31 | 2 | Mat_srv |
| Services 9 | 438.00 | 438.18 | 90.50 | 104.00 | 2 | Mat_srv |
| Services 10 | 440.00 | 440.32 | 88.50 | 211.00 | 2 | Mat_srv |
| Services 11 | 440.00 | 450.00 | 211.00 | 211.41 | 2 | Mat_srv |
| Services 12 | 420.00 | 420.58 | 91.50 | 132.00 | 2 | Carbon |
| Services 13 | 442.00 | 450.00 | 219.00 | 219.26 | 2 | Carbon |
| Services 14 | -440.00 | 440.00 | 146.70 | 147.04 | 1 | Carbon |
| Services 15 | 400.00 | 440.00 | 132.00 | 132.88 | 2 | Mat_srv |
| Services 16 | 428.00 | 428.22 | 126.00 | 132.00 | 2 | Mat_srv |
| Services 17 | 440.00 | 440.70 | 131.00 | 221.00 | 2 | Mat_srv |
| Services 18 | 440.00 | 460.00 | 221.00 | 221.60 | 2 | Mat_srv |
| Services 19 | 445.00 | 445.58 | 140.00 | 223.00 | 2 | Carbon |
| Services 20 | -700.00 | 700.00 | 223.00 | 223.78 | 1 | Carbon |
| Services 21 | -3300.00 | 3330.00 | 230.00 | 231.09 | 1 | Carbon |
| Services 22 | -400.00 | 400.00 | 75.00 | 75.21 | 1 | Carbon |
| Services 23 | 422.00 | 450.00 | 75.00 | 75.21 | 2 | Carbon |

Material composition

| Material | Elements (compound) | Parameters | Comments | Part of pixel | Density (g/cm3) |
| --- | --- | --- | --- | --- | --- |
| Sn | Sn |  |  |  | 7.31 |
| Mat_srv | H | 4 | W | Ladder | 3.2 |
|  | C | 8 |  |  |  |
|  | O | 1 |  |  |  |
|  | Al | 1 |  |  |  |
|  | Si | 4 |  |  |  |
| MatHyb | H | 4 | W | Hybrid | 3.9 |
|  | C | 8 |  |  |  |
|  | O | 1 |  |  |  |
|  | Al | 1 |  |  |  |
|  | Si | 14 |  |  |  |
| MatChips | Al | 1 | W | Electronics | 3.2 |
|  | Si | 8 |  |  |  |
|  | Sn | 1 |  |  |  |
| MatSil | Si |  |  | Silicon crystal |  |
| MatCab | H | 4 | W | Cables | 2.2 |
|  | C | 1 |  |  |  |
|  | Al | 5 |  |  |  |

Pixel Forward

The Pixel forward contains:

- 3 disks

- Services

|  | Z1[mm] | Z2[mm] | Zcenter[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- | --- |
| Disk 1 | 490.00 | 500.00 | 495.00 | 85.00 | 148.00 |
| Disk 2 | 575.00 | 585.00 | 580.00 | 85.00 | 148.00 |
| Disk 3 | 645.00 | 655.00 | 650.00 | 85.00 | 148.00 |

There are two rings of modules on each disk. The coordinates of these rings are in next table

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Ring of modules - disk 1 | 498.18 | 498.82 | 85.00 | 148.00 |
| Ring of modules - disk 1 | 491.68 | 492.32 | 85.00 | 148.00 |
| Ring of modules - disk 2 | 583.47 | 583.82 | 85.00 | 148.00 |
| Ring of modules - disk 2 | 576.68 | 577.32 | 85.00 | 148.00 |
| Ring of modules - disk 3 | 653.18 | 653.82 | 85.00 | 148.00 |
| Ring of modules - disk 3 | 646.68 | 647.32 | 85.00 | 148.00 |

On each ring of modules is situated 24 modules. The dimensions of module parts are in next table:

I use a three dimensional coordinate system (X, Y, Z) for module parts.

The coordinate system is following:

- Axis X – perpendicular to Z-axis, direction – up

- Axis Y – perpendicular to X- and Z-axis, direction - forward

- Axis Z – parallel to beam pipe, direction – right

The shape of module parts is box.

DX, DY, DZ is half of box dimensions.

|  | DX[mm] | DY[mm] | DZ[mm] |
| --- | --- | --- | --- |
| Si motherboard | 31.50 | 9.30 | 0.13 |
| Hybrid | 30.70 | 9.80 | 0.10 |
| Electronics | 30.30 | 11.10 | 0.07 |
| Active zone of pixel | 30.40 | 8.20 | 0.13 |

There are center coordinates for 1st (highest one) module in next table.

|  | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| Si motherboard | 116.50 | 0.00 | 0.00 |
| Hybrid | 116.50 | 0.00 | 0.22 |
| Electronics | 116.50 | 0.00 | -0.22 |
| Active zone of pixel | 116.50 | 0.00 | 0.00 |

Services

There are cable positions for individual subdisks.

| Cables | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Subdisk 1 | 490.8486 | 490.8514 | 160.00 | 175.00 |
| Subdisk 2 | 499.1486 | 499.1514 | 160.00 | 175.00 |
| Subdisk 3 | 575.8486 | 575.8514 | 160.00 | 175.00 |
| Subdisk 4 | 584.1486 | 584.1514 | 160.00 | 175.00 |
| Subdisk 5 | 645.8486 | 645.8514 | 160.00 | 175.00 |
| Subdisk 6 | 654.1486 | 654.1514 | 160.00 | 175.00 |

There are services in next 3 tables.

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Heat Exchanger | 494.39 | 495.61 | 88.10 | 148.00 |
| Heat Exchanger | 579.39 | 580.61 | 88.10 | 148.00 |
| Heat Exchanger | 649.39 | 650.61 | 88.10 | 148.00 |

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| End cap inner part | 494.39 | 495.61 | 87.00 | 88.10 |
| End cap inner part | 579.39 | 580.61 | 87.00 | 88.10 |
| End cap inner part | 649.39 | 650.61 | 87.00 | 88.10 |

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| End cap outer part | 494.22 | 495.78 | 148.00 | 170.00 |
| End cap outer part | 579.22 | 580.78 | 148.00 | 170.00 |
| End cap outer part | 649.22 | 650.78 | 148.00 | 170.00 |

|  | R1[mm] | R2[mm] | Z1[mm] | Z2[mm] | Material |
| --- | --- | --- | --- | --- | --- |
| Services 1 | 175.00 | 180.00 | 495.00 | 495.60 | Mat_srv |
| Services 2 | 175.00 | 180.00 | 580.00 | 580.60 | Mat_srv |
| Services 3 | 175.00 | 180.00 | 650.00 | 650.60 | Mat_srv |
| Services 4 | 180.00 | 180.30 | 446.00 | 539.00 | Carbon |
| Services 5 | 180.00 | 180.30 | 523.00 | 632.00 | Carbon |
| Services 6 | 180.00 | 180.30 | 586.50 | 708.50 | Carbon |
| Services 7 | 177.00 | 180.00 | 490.00 | 492.60 | Carbon |
| Services 8 | 177.00 | 180.00 | 575.00 | 577.60 | Carbon |
| Services 9 | 177.00 | 180.00 | 645.00 | 647.60 | Carbon |
| Services 10 | 181.00 | 188.00 | 495.04 | 495.16 | Carbon |
| Services 11 | 181.00 | 186.00 | 580.04 | 580.16 | Carbon |
| Services 12 | 181.00 | 184.00 | 650.04 | 650.16 | Carbon |
| Services 13 | 188.00 | 188.40 | 491.80 | 669.20 | Mat_srv |
| Services 14 | 186.00 | 186.40 | 534.70 | 712.30 | Mat_srv |
| Services 15 | 184.00 | 184.40 | 570.10 | 747.90 | Mat_srv |
| Services 16 | 182.00 | 182.30 | 1620.80 | 2525.20 | Mat_srv |
| Services 17 | 181.00 | 181.30 | 1621.20 | 2525.80 | Mat_srv |
| Services 18 | 180.00 | 180.30 | 1621.50 | 2526.50 | Mat_srv |
| Services 19 | 189.00 | 205.00 | 500.00 | 504.20 | Mat_srv |
| Services 20 | 189.00 | 205.00 | 585.00 | 589.20 | Mat_srv |
| Services 21 | 189.00 | 205.00 | 655.00 | 659.20 | Mat_srv |
| Services 22 | 86.00 | 86.20 | 498.00 | 624.00 | Carbon |
| Services 23 | 86.00 | 86.20 | 656.20 | 803.80 | Carbon |
| Services 24 | 86.00 | 160.00 | 785.00 | 785.20 | Carbon |
| Services 25 | 165.00 | 180.00 | 785.00 | 785.20 | Carbon |
| Services 26 | 185.00 | 215.00 | 0.00 | 1570.20 | Carbon |

Material composition

| Material | Elements (compound) | Parameters | Comments | Part of P. forward | Density (g/cm3) |
| --- | --- | --- | --- | --- | --- |
| MatDsk | C | 8 | W | Heat exchanger | 1.6 |
|  | H | 4 |  | End cap inner part |  |
|  | O | 1 |  | End cap outer part |  |
|  | Al | 1 |  |  |  |
| Mat_srv | C | 4 | W | Services | 1.03 |
|  | H | 4 |  |  |  |
|  | O | 1 |  |  |  |
|  | Al | 11 |  |  |  |
|  | Be | 4 |  |  |  |
| Mat_cab | O | 2 | W | Cables | 1.6 |
|  | Si | 1 |  |  |  |
| Si | Si | 1 |  | Hybrid |  |
|  |  |  |  | Electronics |  |
|  |  |  |  | Active part |  |
|  |  |  |  | Motherboard |  |

In previous tables it was presented approximation of material composition for each parts. In next tables, there is presented exact element composition for individual blocks of pixel part. But there are presented only dangerous elements in point of view activation. Therefore the element table have to be correlated together with previous material tables.

There is presented dimensions of individual blocks.

| Element | R1 [mm] | R2 [mm] | Z1 [mm] | Z2 [mm] |
| --- | --- | --- | --- | --- |
| B-layer | 45.5 | 74 | -400 | 400 |
| Layer 1 | 83 | 111 | -400 | 400 |
| Layer 2 | 117 | 144 | -400 | 400 |
| Disk 1 | 85 | 148 | 490 | 500 |
| Disk 2 | 85 | 148 | 575 | 585 |
| Disk 3 | 85 | 148 | 645 | 655 |
| B-layer end | 45.5 | 74 | 400 | 442 |
| Layer 1 end | 83 | 111 | 400 | 442 |
| Layer 2 end | 117 | 144 | 400 | 442 |
| Disk 1 cooling connections | 148 | 170 | 490 | 500 |
| Disk 2 cooling connections | 148 | 170 | 575 | 585 |
| Disk 3 cooling connections | 148 | 170 | 645 | 655 |
| Barrel radial services | 45.5 | 170 | 442 | 484 |
| Barrel/disk type 0 services along frame | 170 | 215 | 442 | 700 |
| Outer frame connections | 205 | 215 | 420 | 442 |
| PP0 | 170 | 215 | 700 | 1070 |
| Type 1 services | 170 | 215 | 1070 | 3350 |
| PP1 and end plug | 170 | 215 | 3350 | 3500 |
| Central PST | 227 | 228 | -800 | 800 |
| Pixel to PST to SCT fixations | 228 | 285 | 750 | 800 |
| Forward PST | 227 | 228 | 800 | 3420 |
| Type 2 services | 215 | 2800 | 3420 | 3500 |

There are presented dangerous elements for individual blocks.

| Elements | Element ID | Number | Al | Cu | Ni | Sn | Pb | Ag | Au | Fe |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B-layer | 1 | 1 | 425.0 | 109.7 | 8.7 | 6.9 | 6.4 | 15.8 | 0.2 | 38.6 |
| Layer 1 | 2 | 1 | 587.4 | 187.4 | 10.8 | 11.9 | 11.0 | 27.3 | 0.4 | 40.7 |
| Layer 2 | 3 | 1 | 803.8 | 256.8 | 13.6 | 16.2 | 15.0 | 37.4 | 0.5 | 48.8 |
| Disk 1 | 4 | 2 | 107.6 | 17.3 | 0.4 | 1.2 | 1.1 | 2.7 | 0.0 | 0.0 |
| Disk 2 | 5 | 2 | 107.6 | 17.3 | 0.4 | 1.2 | 1.1 | 2.7 | 0.0 | 0.0 |
| Disk 3 | 6 | 2 | 107.6 | 17.3 | 0.4 | 1.2 | 1.1 | 2.7 | 0.0 | 0.0 |
| B-layer end | 7 | 2 | 3.5 | 14.7 | 17.1 | 0.0 | 0.0 | 0.0 | 0.0 | 15.3 |
| Layer 1 end | 8 | 2 | 6.0 | 25.3 | 29.7 | 0.0 | 0.0 | 0.0 | 0.0 | 27.1 |
| Layer 2 end | 9 | 2 | 8.2 | 34.6 | 39.0 | 0.0 | 0.0 | 0.0 | 0.0 | 27.1 |
| Disk 1 cooling connections | 10 | 2 | 12.4 | 1.6 | 1.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Disk 2 cooling connections | 11 | 2 | 12.4 | 1.6 | 1.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Disk 3 cooling connections | 12 | 2 | 12.4 | 1.6 | 1.6 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Barrel radial services | 13 | 2 | 931.9 | 19.6 | 5.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Barrel/disk type 0 services along frame | 14 | 2 | 4155.7 | 75.8 | 21.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Outer frame connections | 15 | 2 | 14.5 | 0.0 | 8.6 | 0.0 | 0.0 | 0.0 | 0.0 | 53.3 |
| PP0 | 16 | 2 | 640.0 | 599.9 | 0.0 | 31.8 | 16.7 | 0.0 | 0.0 | 0.0 |
| Type 1 services | 17 | 2 | 10149.4 | 133.4 | 0.0 | 0.0 | 0.0 | 186.6 | 0.0 | 0.0 |
| PP1 and end plug | 18 | 2 | 980.9 | 164.8 | 620.6 | 16.3 | 12.1 | 0.0 | 0.5 | 3847.7 |
| Central PST | 19 | 1 | 580.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Pixel to PST to SCT fixations | 20 | 2 | 7.3 | 0.0 | 4.3 | 0.0 | 0.0 | 0.0 | 0.0 | 26.7 |
| Forward PST | 21 | 2 | 1103.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Type 2 services | 22 | 2 | 0.0 | 71063.3 | 8992.3 | 0.0 | 0.0 | 0.0 | 0.0 | 6992.6 |
| GRAN TOTAL |  |  | 39116.8 | 144930.4 | 19523.1 | 138.3 | 96.3 | 469.7 | 2.4 | 22107.7 |

| Elements | Element ID | Number | Cr | In | Ru | Pd | Mg | Mo | Ti |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B-layer | 1 | 1 | 16.3 | 0.0 | 0.1 | 0.0 | 1.0 | 1.9 | 9.1 |
| Layer 1 | 2 | 1 | 17.7 | 0.1 | 0.1 | 0.0 | 1.7 | 2.0 | 15.8 |
| Layer 2 | 3 | 1 | 21.4 | 0.1 | 0.1 | 0.1 | 2.4 | 2.3 | 21.6 |
| Disk 1 | 4 | 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.5 |
| Disk 2 | 5 | 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.5 |
| Disk 3 | 6 | 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.5 |
| B-layer end | 7 | 2 | 6.2 | 0.7 | 0.0 | 0.0 | 0.0 | 0.7 | 0.0 |
| Layer 1 end | 8 | 2 | 10.9 | 1.2 | 0.0 | 0.0 | 0.0 | 1.3 | 0.0 |
| Layer 2 end | 9 | 2 | 10.9 | 1.7 | 0.0 | 0.0 | 0.0 | 1.3 | 0.0 |
| Disk 1 cooling connections | 10 | 2 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Disk 2 cooling connections | 11 | 2 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Disk 3 cooling connections | 12 | 2 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Barrel radial services | 13 | 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Barrel/disk type 0 services along frame | 14 | 2 | 0.0 | 0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Outer frame connections | 15 | 2 | 21.5 | 0.0 | 0.0 | 0.0 | 0.0 | 2.6 | 0.0 |
| PP0 | 16 | 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Type 1 services | 17 | 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| PP1 and end plug | 18 | 2 | 1551.5 | 0.0 | 0.0 | 0.0 | 0.0 | 186.2 | 0.0 |
| Central PST | 19 | 1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Pixel to PST to SCT fixations | 20 | 2 | 10.8 | 0.0 | 0.0 | 0.0 | 0.0 | 1.3 | 0.0 |
| Forward PST | 21 | 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Type 2 services | 22 | 2 | 2819.6 | 0.0 | 0.0 | 0.0 | 0.0 | 338.4 | 0.0 |
| GRAN TOTAL |  |  | 8918.2 | 8.9 | 0.4 | 0.1 | 5.1 | 1069.7 | 55.8 |