# Geometry of SCT barrel

The SCT barrel consists from:

- Thermal shields and barrel interlink

- 4  SCT barrel with modules

- Services (cables, cooling pipes, etc.)

The position of thermal shields and barrel interlink is in next two tables

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Thermal shield 1 | -754.00 | 754.00 | 548.3 | 549.00 |
| Thermal shield 2 | 754.00 | 790.00 | 540.00 | 549.00 |
| Thermal shield 3 | 724.00 | 754.00 | 540.00 | 548.30 |
| Thermal shield 4 | 233.00 | 263.00 | 540.00 | 548.30 |
| Thermal shield 5 | 788.20 | 790.00 | 250.00 | 540.00 |
| Thermal shield 6 | -770.00 | 770.00 | 250.00 | 250.175 |

|  | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Barrel interlink | 787.00 | 788.20 | 260.00 | 500.00 |

The barrel is compound from:

- Support cylinder with special material at the end

- Modules

In first cell of the table is number of modules on barrel. The coordinates for individual barrels are in next 4 tables

| SCT barrel 3 - 12*32 modules | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Support cylinder | -782.94 | 782.94 | 278.00 | 284.00 |
| Close out at the end of barrel | 782.94 | 783.20 | 278.00 | 284.00 |
| End flange | 783.20 | 787.00 | 258.00 | 284.00 |

| SCT barrel 4 (12*40 modules) | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Support cylinder | -782.94 | 782.94 | 349.00 | 355.00 |
| Close out at the end of barrel | 782.94 | 783.20 | 349.00 | 355.00 |
| End flange | 783.20 | 787.00 | 325.00 | 355.00 |

| SCT barrel 5 (12*48 modules) | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Support cylinder | -782.94 | 782.94 | 421.00 | 427.00 |
| Close out at the end of barrel | 782.94 | 783.20 | 421.00 | 427.00 |
| End flange | 783.20 | 787.00 | 395.00 | 427.00 |

| SCT barrel 6 (12*56 modules) | Z1[mm] | Z2[mm] | R1[mm] | R2[mm] |
| --- | --- | --- | --- | --- |
| Support cylinder | -782.94 | 782.94 | 492.00 | 498.00 |
| Close out at the end of barrel | 782.94 | 783.20 | 492.00 | 498.00 |
| End flange | 783.20 | 787.00 | 466.00 | 498.00 |

The center coordinates of module holders (the physical center of module) are in next table. The Z-coordinates are same for all barrels.

| Center coordinate of module holder | Z[mm] |
| --- | --- |
| module 12 | 679.05 |
| module 11 | 550.454 |
| module 10 | 434.277 |
| module 9 | 306.533 |
| module 8 | 184.032 |
| module 7 | 60.288 |
| module 6 | -63.454 |
| module 5 | -187.094 |
| module 4 | -310.837 |
| module 3 | -432.141 |
| module 2 | -560.099 |
| module 1 | -679.05 |

I use a three dimensional coordinate system (X, Y, Z) for module parts.

The coordinate system is following:

- Axis X – perpendicular to Z-axis, direction – up

- Axis Y – perpendicular to X- and Z-axis, direction - forward

- Axis Z – parallel to beam pipe, direction – right

The shape of module parts is box except a cooling pipe (tube).

DX, DY, DZ is half of box dimensions, R1 is inner radius, R2 outer radius of cooling pipe

The module consists of:

- Silicon part

- Hybrid

- Baseboard

- Power tape

- Cooling pipe (it is common for 12 detectors)

- Box to hold module

- Cooling block

- Dogleg

The dimensions of individual module parts are in next table

| Module | DX[mm], R1[mm] | DY[mm], R2[mm] | DZ[mm] | Shape |
| --- | --- | --- | --- | --- |
| Silicon part | 0.14 | 31.78 | 64.00 | box |
| Hybrid | 0.37 | 37.30 | 12.50 | box |
| Baseboard | 0.25 | 43.80 | 29.65 | box |
| Power tape | 0.11 | 10.50 | 52.73 | box |
| Cooling pipe | 0.00 | 2.10 | 787.00 | tube |
| Box to hold module | 0.8 | 7.50 | 30.00 | box |
| Cooling block | 1.00 | 2.50 | 30.00 | box |
| Dogleg | 0.13 | 27.50 | 15.00 | box |

The real coordinates of module 1 (the center coordinates) on disk 3 are written in next table

| Module | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| Silicon part | 299.04425 | -4.78 | -679.05 |
| Silicon part | 298.95575 | -4.78 | -679.05 |
| Hybrid | 299.095 | -4.78 | -661.55 |
| Hybrid | 298.905 | -4.78 | -661.55 |
| Baseboard | 299.00 | -4.78 | -666.55 |
| Power tape | 285.17 | -14.00 | -734.27 |
| Cooling pipe | 300.841 | -21.64 | 0.00 |
| box to hold module | 284.80 | 15.00 | -666.55 |
| Cooling block | 303.85 | -23.15 | -666.55 |
| Dogleg | 296.11 | 0.00 | -666.55 |

## Services

The meanings of next table is following:

- SCT pipes – pipe that takes away cooling fluid

- Cables, alignment tubes – cables from barrels

- SCT services – cables and cooling pipes from barrels to PPB1

- SCT services – cables and cooling pipes from PPB1 to PPB2 ( the coordinates are just until end of the  SCT forward region)

|  | R1[mm] | R2[mm] | Z1[mm] | Z2[mm] |
| --- | --- | --- | --- | --- |
| SCT pipes | 300.841 | 500.00 | 780.00 | 785.00 |
| Cables, alignment tubes | 260.00 | 500.00 | 765.00 | 775.00 |
| SCT services | 500.00 | 1140.00 | 780.00 | 798.40 |
| SCT services | 1123.00 | 1150.00 | 798.40 | 3430.00 |

The exact position of patch panels is unknown

Cables

There are two types of cables between modules and PP2

- Type I  -  Module -> PPB1

- Type IIa (IIb) - PPB1 -> PP2

Type I - kapton tape with 0.75 m thick aluminum conductors

Type II - copper conductor plated by 1.6 m layer of Ni (Ag)

The properties of these cables are summarized in next table

|  | Type I | Type IIa | Type IIb |
| --- | --- | --- | --- |
| Type | Tape | Cable | Cable |
| Outer diameter[mm] |  | 6.00 | 7.06 |
| Area[mm2] |  | 28.26 | 39.127226 |
| Total number barrel | 1056 | 1056 | 1056 |
| Total number forward | 988 | 988 | 988 |
| Number of cores |  | 17 | 36 |
| Copper content[mm2] |  | 3.587 | 7.05 |
| Ni (Ag) content[mm2] |  | 0.131 | 0.2738 |
| AWG 20 |  | 4 | 8 |
| AWG24 |  | 1 | 1 |
| AWG28 |  | 11 | 28 |
| AWG30 |  | 2 | 0 |

# Material composition of SCT barrel

| Material | Elements (compound) | Parameters | Comments | Part of SCT | Density (g/cm3) |
| --- | --- | --- | --- | --- | --- |
| ### TSC | CFRP | 0.367 | Weight percentage | Thermal shield 1 | 1.97 |
|  | Al | 0.495 |  |  |  |
|  | C | 0.022 |  |  |  |
|  | F | 0.094 |  |  |  |
|  | Kapton | 0.022 |  |  |  |
| TSE | CFRP | 0.508 | Weight percentage | Thermal shield 2 | 0.795 |
|  | Al | 0.193 |  |  |  |
|  | C | 0.046 |  |  |  |
|  | F | 0.196 |  |  |  |
|  | Airex | 0.057 |  |  |  |
| TSP | CFRP | 0.9 | Weight percentage | Thermal shield 5 | 0.569 |
|  | Al | 0.095 |  |  |  |
|  | Airex | 0.005 |  |  |  |
| EMI | Al | 0.588 | Weight percentage | Thermal shield 6 | 1.97 |
|  | Kapton | 0.412 |  |  |  |
| Airex 82 | Polystyrene |  |  | Thermal shield 3, 4 | 0.62 |
| CuKapton | Cu | 0.863 |  | Dogleg | 5.19 |
|  | Kapton | 0.137 |  |  |  |
| C-fibers | C | 1 |  | Supports cylinders | 0.2 |
| Flange | Kapton |  |  | Barrel interlink, End flange of barrels | 1.6 |
| CN107 | Cu | 0.7 |  | Close out at the end of barrels | 8.95 |
|  | Ni | 0.3 |  |  |  |

The exact composition of module is in the next table

|  | H | Be | B 10 | B 11 | C | N | O |
| --- | --- | --- | --- | --- | --- | --- | --- |
| #### Si sensors and adhesive dimensions | 0.001 | 0.00 | 0.002 | 0.008 | 0.037 | 0.017 | 0.011 |
| Baseboard and BeO facings dimension | 0.008 | 0.597 | 0.011 | 0.043 | 4.672 | 0.092 | 1.119 |
| ASICs and adhesive dimension | 0.004 | 0.00 | 0.005 | 0.022 | 0.099 | 0.045 | 0.037 |
| Cooling pipe with coolant dimensions | 0.00 | 0.00 | 0.00 | 0.00 | 0.247 | 0.00 | 0.00 |
| Power tape dimensions | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Hybrid with passive components dimensions | 0.034 | 0.00 | 0.039 | 0.146 | 3.024 | 0.311 | 0.442 |
| TOTAL (per 1 detector) | 0.047 | 0.597 | 0.057 | 0.219 | 8.079 | 0.465 | 1.609 |
| Si sensors and adhesive dimensions | F | Al | Si | Ni | Cu | Ag | Sn |
| Baseboard and BeO facings dimension | 0.00 | 0.00 | 10.812 | 0.00 | 0.00 | 0.00 | 0.00 |
| ASICs and adhesive dimension | 0.00 | 0.118 | 0.742 | 0.00 | 0.00 | 0.00 | 0.00 |
| Cooling pipe with coolant dimensions | 0.00 | 0.028 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Power tape dimensions | 3.462 | 1.116 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Hybrid with passive components dimensions | 0.00 | 0.78 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| TOTAL (per 1 detector) | 0.00 | 0.393 | 0.187 | 0.025 | 1.501 | 0.001 | 0.049 |
|  | 3.462 | 2.435 | 11.741 | 0.025 | 1.501 | 0.001 | 0.049 |
|  | Au | Pb |  |  |  |  |  |
| Si sensors and adhesive dimensions | 0.00 | 0.00 |  |  |  |  |  |
| Baseboard and BeO facings dimension | 0.00 | 0.00 |  |  |  |  |  |
| ASICs and adhesive dimension | 0.00 | 0.00 |  |  |  |  |  |
| Cooling pipe with coolant dimensions | 0.00 | 0.00 |  |  |  |  |  |
| Power tape dimensions | 0.00 | 0.00 |  |  |  |  |  |
| Hybrid with passive components dimensions | 0.011 | 0.073 |  |  |  |  |  |
| TOTAL (per 1 detector) | 0.011 | 0.073 |  |  |  |  |  |

Geometry and material composition of SCT forward

The SCT forward consists from:

- Thermal shields and outer support frame

- 9 disk (ring of modules, support system, etc.)

- Services (cables, cooling pipes, etc.)

The outer support frame is situated in

|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| --- | --- | --- | --- | --- |
| Outer support frame | 823.00 | 2757.00 | 572.00 | 582.00 |

There are many thermal shields in SCT forward region, the positions of thermal shields are written in next table

|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| --- | --- | --- | --- | --- |
| Thermal shield 1 | 811.00 | 812.00 | 251.00 | 610.00 |
| Thermal shield 2 | 820.00 | 820.20 | 500.00 | 590.00 |
| Thermal shield 3 | 820.50 | 840.00 | 582.00 | 599.00 |
| Thermal shield 4 | 820.00 | 825.00 | 599.00 | 610.00 |
| Thermal shield 5 | 825.00 | 2733.00 | 599.00 | 610.00 |
| Thermal shield 6 | 825.00 | 2733.00 | 610.00 | 610.85 |
| Thermal shield 7 | 2749.00 | 2749.23 | 258.00 | 572.00 |
| Thermal shield 8 | 2749.23 | 2757.23 | 258.00 | 572.00 |
| Thermal shield 9 | 2786.00 | 2787.00 | 258.00 | 615.00 |
| Thermal shield 10 | 2758.00 | 2785.00 | 258.00 | 291.50 |
| Thermal shield 11 | 813.00 | 2787.00 | 255.50 | 258.00 |
| Thermal shield 12 | 813.00 | 818.50 | 258.00 | 305.00 |

The SCT forward region consists of 9 disks. The coordinates of individual disks are written in next table. The column “Z centre” is center of  support disk that holds ring of modules.

|  | Zmin[mm] | Zmax[mm] | # Z centre[mm] | Rmin[mm] | Rmax[mm] |
| --- | --- | --- | --- | --- | --- |
| SCT disk 1 | 828.50 | 868.50 | 848.50 | 267.00 | 567.00 |
| SCT disk 2 | 912.00 | 952.00 | 932.00 | 267.00 | 567.00 |
| SCT disk 3 | 1062.00 | 1102.00 | 1082.00 | 267.00 | 567.00 |
| SCT disk 4 | 1240.00 | 1280.00 | 1260.00 | 267.00 | 567.00 |
| SCT disk 5 | 1355.00 | 1395.00 | 1375.00 | 267.00 | 567.00 |
| SCT disk 6 | 1725.00 | 1765.00 | 1745.00 | 267.00 | 567.00 |
| SCT disk 7 | 2050.00 | 2090.00 | 2070.00 | 267.00 | 567.00 |
| SCT disk 8 | 2440.00 | 2480.00 | 2460.00 | 374.00 | 567.00 |
| SCT disk 9 | 2705.00 | 2745.00 | 2725.00 | 374.00 | 567.00 |

The coordinates of cooling pipes and power tapes for individual disk are presented in next two tables

|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| --- | --- | --- | --- | --- |
| Cooling pipe – Disk 1 | 848.50 | 2757.00 | 583.90 | 583.94946 |
| Cooling pipe – Disk 2 | 932.00 | 2757.00 | 583.94946 | 584.02364 |
| Cooling pipe – Disk 3 | 1082.00 | 2757.00 | 584.02364 | 584.09782 |
| Cooling pipe – Disk 4 | 1260.00 | 2757.00 | 584.09782 | 584.172 |
| Cooling pipe – Disk 5 | 1375.00 | 2757.00 | 584.172 | 584.24618 |
| Cooling pipe – Disk 6 | 1745.00 | 2757.00 | 584.24618 | 584.32036 |
| Cooling pipe – Disk 7 | 2070.00 | 2757.00 | 584.32036 | 584.36982 |
| Cooling pipe – Disk 8 | 2460.00 | 2757.00 | 584.36982 | 584.41928 |
| Cooling pipe – Disk 9 | 2725.00 | 2757.00 | 584.41928 | 584.44399 |

|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| --- | --- | --- | --- | --- |
| Power tape Disk 1 | 845.00 | 2757.00 | 598.50 | 598.5494 |
| Power tape Disk 2 | 932.00 | 2757.00 | 598.5494 | 598.6194 |
| Power tape Disk 3 | 848.50 | 2757.00 | 598.6194 | 598.6894 |
| Power tape Disk 4 | 1082.00 | 2757.00 | 598.6894 | 598.7594 |
| Power tape Disk 5 | 1260.00 | 2757.00 | 598.7594 | 598.8294 |
| Power tape Disk 6 | 1375.00 | 2757.00 | 598.8294 | 598.8994 |
| Power ape Disk 7 | 1745.00 | 2757.00 | 598.8994 | 598.9488 |
| Power tape Disk 8 | 2070.00 | 2757.00 | 598.9488 | 598.9982 |
| Power tape Disk 9 | 2460.00 | 2757.00 | 598.9982 | 599.0261 |

The disks are held by support system. There are two types of support system:

- Support system for disks number 1,..,7

- Support system for disks number 8 and 9

The coordinates of support system are in following table (the Z-coordinates are relative, the Z=0 is taken in the middle of the each disk, and they have to be recomputed to real position, for example - disk 1: Zmin = 844.5 mm, Zmax = 852.5 mm)

|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| --- | --- | --- | --- | --- |
| Support disk for the disks 1-7 | -4.00 | 4.00 | 267.00 | 567.00 |
| Support disk for the disks 8,9 | -4.00 | 4.00 | 374.00 | 567.00 |

The each disk has own patch panel. There are three types of patch panels:

- Patch panel for disks number 1, 3, 4, 5 and 6

- Patch panel for disks number 2, 7 and 8

- Patch panel for disks number 9

The coordinates of patch panels are in next table (the Z-coordinates are relative, the Z=0 is taken in the middle of the each disk and they have to be recomputed to real coordinates, for example - disk 1: Zmin = 832.5 mm, Zmax = 844.5 mm)

|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| --- | --- | --- | --- | --- |
| Patch panel 1,3,4,5,6 | -16.00 | -4.00 | 557.00 | 567.00 |
| Patch panel 2,7,8 | -12.00 | -4.00 | 557.00 | 567.00 |
| Patch panel 9 | -9.00 | -4.00 | 557.00 | 567.00 |

There are 4 types of module ring in SCT forward region. The types of ring and positions are written in next table. The detail information about rings follows after the table.

The meaning of columns in table is following:

- Number of rings – number of module rings on appropriate disk

- List of rings – type of module rings

- Side of disk to mount rings – side of disk, where the ring of modules is situated (-1 – the ring of modules  is situated left from the center of disk, +1 – the ring of modules is situated right from the center of the disk)

|  | Number of rings | List of rings | Side of disk to mount rings |
| --- | --- | --- | --- |
| Disk 1 | 2 | 1,2 | {-1,+1} |
| Disk 2 | 3 | 1,2,4 | {-1,+1,-1} |
| Disk 3 | 3 | 1,2,4 | {-1,+1,-1} |
| Disk 4 | 3 | 1,2,4 | {-1,+1,-1} |
| Disk 5 | 3 | 1,2,4 | {-1,+1,-1} |
| Disk 6 | 3 | 1,2,4 | {-1,+1,-1} |
| Disk 7 | 2 | 1,2 | {-1,+1} |
| Disk 8 | 2 | 1,3 | {-1,+1} |
| Disk 9 | 1 | 1 | {-1} |

The next four tables describe parts of individual rings. The meaning of rows is following:

- No. of modules – number of modules that are situated on the ring of modules

- Cooling block – coordinates of cooling block that is situated on the disk (Z-coordinates are relative,  Z=0 is taken in the middle of the each disk and                        they have to be recomputed for individual disks to real coordinates, for example: disk 1:  Zmin = 844.206 mm, Zmax = 844.5 mm)

- Power tape – coordinates of power tape that is positioned on the disk (Z-coordinates are relative, the Z=0 is taken in the middle of the each disk and                        they have to be recomputed for individual disks to real coordinates, for example: disk 1: Zmin = 844.128 mm, Zmax = 844.206 mm)

- Ring of modules – coordinates of ring of modules that is positioned on the disk

! The Z-coordinates of ring of modules have to be multiplied by appropriate coefficient from column “Side of disk to mount rings” from previous table!

(Z-coordinates are relative, Z=0 is taken in the middle of the each disk and they have to be recomputed for individual disks to real coordinates, for example: disk 1, first ring of modules: Zmin =830.478 mm, Zmax = 833.022 mm)

- Module – in next three rows are dimensions of  individual module parts

I use a three dimensional coordinate system (X, Y, Z) for the module parts.

The coordinate system is following:

- Axis X – perpendicular to Z-axis, direction – up

- Axis Y – perpendicular to X- and Z-axis, direction - forward

- Axis Z – parallel to beam pipe, direction – right

The shape of individual module parts is box. DX, DY, DY1, DY2, DZ is half of box width.

| Ring 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. of modules | 52 |  |  |  |
|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| Cooling block | -4.294 | -4.00 | 383.00 | 567.00 |
| Power tape | -4.372 | -4.294 | 398.315 | 557.00 |
| Ring of module | 15.478 | 18.022 | 398.315 | 568.41 |
| Ring of module | 10.778 | 13.322 | 398.315 | 568.41 |
| Module | Dx[mm] | Dy,Dy1[mm] | Dy2[mm] | Dz[mm] |
| Hybrid | 17.50 | 20.00 | 38.50 | 1.27 |
| Cooling between wafers | 66.07 | 20.00 | - | 0.25 |
| Si sensors | 61.62 | 28.13 | 36.05 | 0.14 |

There are coordinates (the center coordinates) of 1. module (the highest one) on ring of modules in next table.

The Z-coordinate (relative) is taken from the middle of the ring of modules (Z=0), X- and Y-coordinates are real

(For example: ring of modules 1, module 1: Hybrid: Z1= 830.48, Z2= 833.02, X1= 398.315, X2=433.315, etc.)

| Module | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| Hybrid | 415.815 | 0.00 | 0.00 |
| Si sensors | 499.38 | 0.00 | 0.49 |
| Cooling between wafers | 499.38 | 0.00 | 0.00 |
| Si sensors | 499.38 | 0.00 | -0.49 |

| Ring 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. of modules | 40 |  |  |  |
|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| Cooling block | -4.272 | -4.00 | 329.00 | 508.00 |
| Power tape | -4.35 | -4.272 | 497.744 | 557.7 |
| Ring of alternate module | 15.478 | 18.022 | 332.15 | 497.744 |
| Ring of alternate module | 10.778 | 13.322 | 332.15 | 497.744 |
| Module | Dx[mm] | Dy,Dy1[mm] | Dy2[mm] | Dz[mm] |
| Hybrid | 17.50 | 38.50 | 2.00 | 1.27 |
| Cooling between wafers | 64.30 | 20.00 | - | 0.15 |
| Si sensors | 59.85 | 27.74 | 37.60 | 0.14 |

| Module | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| Hybrid | 478.25 | 0.00 | 0.00 |
| Si sensors | 396.45 | 0.00 | 0.49 |
| Cooling between wafers | 396.45 | 0.00 | 0.00 |
| Si sensors | 396.45 | 0.00 | -0.49 |

| Ring 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. of modules | 40 |  |  |  |
|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| Cooling block | -4.272 | -4.00 | 425.00 | 508.00 |
| Power tape | -4.489 | -4.272 | 498.685 | 557.70 |
| Ring of alternate module | 15.478 | 18.022 | 397.375 | 498.684 |
| Ring of alternate module | 10.778 | 13.322 | 397.375 | 498.684 |
| Module | Dx[mm] | Dy,Dy1[mm] | Dy2[mm] | Dz[mm] |
| Hybrid | 17.50 | 20.00 | 38.5 | 1.27 |
| Cooling between wafers | 66.07 | 20.00 | - | 0.25 |
| Si sensors | 61.62 | 28.13 | 36.05 | 0.14 |

| Module | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| Hybrid | 429.06 | 0.00 | 0.00 |
| Si sensors | 478.25 | 0.00 | 0.49 |
| Cooling between wafers | 478.25 | 0.00 | 0.00 |
| Si sensors | 478.25 | 0.00 | -0.49 |

| Ring 4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. of modules | 40 |  |  |  |
|  | Zmin[mm] | Zmax[mm] | Rmin[mm] | Rmax[mm] |
| Cooling block | -4.50 | -4.00 | 283.00 | 379.00 |
| Power tape | -4.579 | -4.501 | 377.351 | 557.70 |
| Ring of alternate module | 15.478 | 18.022 | 269.55 | 377.35 |
| Ring of alternate module | 10.778 | 13.322 | 269.55 | 377.35 |
| Module | Dx[mm] | Dy,Dy1[mm] | Dy2[mm] | Dz[mm] |
| Hybrid | 17.50 | 38.50 | 20.00 | 1.27 |
| Cooling between wafers | 35.00 | 20.00 | - | 0.15 |
| Si sensors | 30.55 | 22.79 | 27.87 | 0.13 |

| Module | X[mm] | Y[mm] | Z[mm] |
| --- | --- | --- | --- |
| Hybrid | 357.05 | 0.00 | 0.00 |
| Si sensors | 304.55 | 0.00 | 0.48 |
| Cooling between wafers | 304.55 | 0.00 | 0.00 |
| Si sensors | 304.55 | 0.00 | -0.49 |

Cables

There are two types of cables between modules and PP2

- Type I  -  Module -> PPB1

- Type IIa (IIb) - PP2->PPB1

Type I - kapton tape with 0.75 m thick aluminum conductors

Type II - copper conductor plated by 1.6 m layer of Ni (Ag)

The properties of these cables are summarized in next table

|  | Type I | Type IIa | Type IIb |
| --- | --- | --- | --- |
| Type | Tape | Cable | Cable |
| Outer diameter[mm] |  | 6.00 | 7.06 |
| Area[mm2] |  | 28.26 | 39.127226 |
| Total number barrel | 1056 | 1056 | 1056 |
| Total number forward | 988 | 988 | 988 |
| Number of cores |  | 17 | 36 |
| Copper content[mm2] |  | 3.587 | 7.05 |
| Ni (Ag) content[mm2] |  | 0.131 | 0.2738 |
| AWG 20 |  | 4 | 8 |
| AWG24 |  | 1 | 1 |
| AWG28 |  | 11 | 28 |
| AWG30 |  | 2 | 0 |

## Material composition of SCT forward

| Material name | Elements | Parameters | Density (g/cm3) | Comments | Part of TRT |
| --- | --- | --- | --- | --- | --- |
| ### C-fibers | C | 1 | 0.145 |  | Outer frame support |
| ### Col_mixture | H | 0.092 | 1.52 | Weight percentage | Cooling tubes with coolant |
|  | Al | 0.366 |  |  |  |
|  | O | 0.542 |  |  |  |
| ### Pow_tapes | Al | 1 | 2.70 |  | Power tapes |
| Kapton-Al alloy |  |  | 0.95 | Front wing heater | Thermal shield 1 |
| ### Aluminum | Al | 1 |  | Heat spreaders | Thermal shield 2, 6, 7, 9 |
| ### Polystyren |  |  | 0.0019 |  | Thermal shield 3, 5, 8 |
| ### Polyethylen | CH2 | 1 | 9.29 |  | Thermal shield 4 |
| ### CFRP | C | 1 | 1.39 |  | Thermal shield 10, 11, 12 |
| C_sup_disk | C | 1 | 0.1184 |  | Support disk for the wheels |
| Patch_panel | Al | 1 | 2.70 | Not in final version | Patch panel |