# TRT geometry and material composition

TRT geometry

TRT detectors are divided into two groups:

- TRT Barrel

- TRT End Cap wheels

TRT End Cap wheels are further divided into three groups:

- TRT A

- TRT B

- TRT C

| TRT | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| TRT barrel | 558.00 | 1073.00 | 0.00 | 712.75 |
| TRT A | 620.00 | 1076.50 | 825.75 | 1707.25 |
| TRT B | 620.00 | 1076.50 | 1719.75 | 2716.25 |
| TRT C | 460.00 | 1034.00 | 2815.74 | 3406.25 |

- TRT Barrel

The TRT barrel has 4 main parts:

- Inner support

- Detector part

- Outer support

- Electronics

|  | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Inner support | 558.00 | 560.00 | 0.00 | 712.75 |
| Detector part | 560.00 | 1070.00 | 0.00 | 712.75 |
| Outer support | 1070.00 | 1073.00 | 0.00 | 712.75 |
| Electronics | 560.00 | 1070.00 | 712.75 | 771.75 |

The detector part is made up of  modules containing straws running parallel to the beam. Each module (except last one) consists from 4 layers of straws. Each layer contains definite number of straws (see table below)

|  | Number of layer | Straws per layer | Comments |  |  |
| --- | --- | --- | --- | --- | --- |
| Module 1-4 | 4 | 539 |  |  |  |
| Module 5- 8 | 4 | 555 |  |  |  |
| Module 9- 12 | 4 | 580 | Modules 1-9 are active from z=403 mm |  |  |  |  |
| Module 13- 16 | 4 | 605 |  |  |  |
| Module 17-20 | 4 | 631 |  |  |  |
| Module 21-24 | 4 | 656 | 1inactive - 21 |  |  |
| Module 25-28 | 4 | 681 |  |  |  |
| Module 29-32 | 4 | 706 |  |  |  |
| Module 33-36 | 4 | 731 |  |  |  |
| Module 37-40 | 4 | 756 |  |  |  |
| Module 41-44 | 4 | 781 |  |  |  |
| Module 45-48 | 4 | 806 | 1 inactive - 45 |  |  |
| Module 49-52 | 4 | 832 |  |  |  |
| Module 53-56 | 4 | 857 |  |  |  |
| Module 57-60 | 4 | 882 |  |  |  |
| Module 61- 64 | 4 | 907 |  |  |  |
| Module 65-68 | 4 | 932 |  |  |  |
| Module 69-72 | 4 | 957 |  |  |  |
| Module 72-75 | 3 | 982 |  |  |  |

The thickness (in R coordinate) of individual modules is 27.2 mm. Each module is segmented into 4 layers. The thickness of one layer is 6.8 mm.

The dimensions of Module 1-4 are in next table

| Module 1-4 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Layer 1 | 560.00 | 566.80 | 0.00 | 712.75 |
| Layer 2 | 566.80 | 573.60 | 0.00 | 712.75 |
| Layer 3 | 573.60 | 580.40 | 0.00 | 712.75 |
| Layer 4 | 580.40 | 587.20 | 0.00 | 712.75 |

Each layer contains definite number of straws. The straws are situated regularly in the layer. The rest space of layer is filled by radiator material.

- Straw (inside) – tube with radius 2 mm

- Straw wall – tube with inner radius 2 mm and outer radius 2.085 mm

- Cooling gas – tube with inner radius 2.085 mm and outer radius 2.4 mm

- Radiator – fills the rest of layer

The dimensions for layer 1 are shown in next table (R-coordinates are relative, the R=0 is situated in the middle of layer 1 (in R coordinate), it corresponds to R=563.4 mm in real coordinate)

| Layer 1 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Radiator | 2.40 | 3.40 | 0.00 | 712.75 |
| Cooling gas | 2.085 | 2.40 | 0.00 | 712.75 |
| Straw wall | 2.00 | 2.085 | 0.00 | 712.75 |
| Straw (inside) | 0.00 | 2.00 | 0.00 | 712.75 |

There are two kinds of straws (inside) in barrel:

- Straw (inside) in layer 1-9

- Straw (inside)  in layer 10-75 ( there are two inactive (without straws) layers (21, 45) that are filled  with carbon (see material composition table)

Straw (inside) in layer 1-9 consist of:

- Wire – tube with radius 0.0155 mm

- Mid-straw twister – tube ( inner radius = 0.0155mm, outer  radius = 0.2 mm, length = 8.12 mm)

- Dead region – tube (inner radius = 0.0155 mm, outer radius = 0.2 mm, length =391.88 mm )

- Dead region – tube (inner radius = 0.0155 mm, outer radius = 0.2 mm, length = 3 mm)

- Gas – tube (inner radius = 0.0155 mm, outer radius = 0.2 mm, length = 306.75 mm)

- Dead region – tube (inner radius = 0.0155 mm, outer radius = 0.2 mm, length = 3 mm)

The dimension of straw (inside) in layers 1-9 are summarized in next table (R-coordinates are relative, the R=0 is situated in the center of corresponding layer, for example 1. layer: R=0.00 corresponds to R=563.4 mm)

| Straw (inside) | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Wire | 0.00 | 0.015 | 0.00 | 712.75 |
| Mid-straw twister | 0.015 | 2.00 | 0.00 | 8.12 |
| Dead region | 0.015 | 2.00 | 8.12 | 400.00 |
| Dead region | 0.015 | 2.00 | 400.00 | 403.00 |
| Gas | 0.015 | 2.00 | 403.00 | 709.75 |
| Dead region | 0.015 | 2.00 | 709.75 | 712.75 |

Straw (inside) in layer 10-75

- Wire – tube with radius 0.0155 mm

- Mid-straw twister – tube ( inner radius = 0.0155mm, outer  radius = 0.2 mm, length = 8.12 mm)

- Dead region – tube (inner radius = 0.0155 mm, outer radius = 0.2 mm, length = 3 mm)

- Gas – tube (inner radius = 0.0155 mm, outer radius = 0.2 mm, length = 698.63 mm)

- Dead region – tube (inner radius = 0.0155 mm, outer radius = 0.2 mm, length = 3 mm)

The dimension of straw (inside) in layers 10-75 are summarized in next table (R-coordinates are relative, the R=0 is situated in the center of corresponding layer  (in R coordinate), for 10. layer, it corresponds to R=632.4 mm)

| Straw (inside) | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Wire | 0.00 | 0.015 | 0.00 | 712.75 |
| Mid-straw twister | 0.015 | 2.00 | 0.00 | 8.12 |
| Dead region | 0.015 | 2.00 | 8.12 | 11.12 |
| Gas | 0.015 | 2.00 | 11.12 | 709.75 |
| Dead region | 0.015 | 2.00 | 709.75 | 712.75 |

- TRT A

TRT A consists of:

- C02 Manifold

- Detector part

- CO2 Manifold

|  | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| CO2 Manifold | 620.00 | 1076.50 | 825.75 | 834.75 |
| Detector part | 620.00 | 1076.50 | 834.75 | 1698.25 |
| CO2 Manifold | 620.00 | 1076.50 | 1698.25 | 1707.25 |

The detector part is divided into:

- 6 TRT parts

- 5 Heat exchangers

The coordinates of TRT parts and heat exchangers are in next table

| Detector part | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| TRT 1 | 620.00 | 1076.50 | 834.75 | 970.75 |
| Heat exchanger | 620.00 | 1027.70 | 970.75 | 980.25 |
| TRT 2 | 620.00 | 1076.50 | 980.25 | 1116.25 |
| Heat exchanger | 620.00 | 1027.70 | 1116.25 | 1125.75 |
| TRT 3 | 620.00 | 1076.50 | 1125.75 | 1261.75 |
| Heat exchanger | 620.00 | 1027.70 | 1261.75 | 1271.25 |
| TRT 4 | 620.00 | 1076.50 | 1271.25 | 1407.25 |
| Heat exchanger | 620.00 | 1027.70 | 1407.25 | 1416.75 |
| TRT 5 | 620.00 | 1076.50 | 1416.75 | 1552.75 |
| Heat exchanger | 620.00 | 1027.70 | 1552.75 | 1562.25 |
| TRT 6 | 620.00 | 1076.50 | 1562.25 | 1698.25 |

Each TRT part is composed from:

- Inner support

- Active part

- Outer support

- Electronics

The coordinates of TRT 1 are written in next tables (the R-coordinates are valid for all TRT parts)

| TRT 1 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Inner support | 620.00 | 643.70 | 834.75 | 970.75 |
| Active part | 643.70 | 1004.00 | 834.75 | 970.75 |
| Outer support | 1004.00 | 1027.70 | 834.75 | 970.75 |
| Electronics | 1027.70 | 1076.50 | 834.75 | 970.75 |

The active part  is segmented into 16 layers. Each layer consists 768 straws. The straws are situated regularly in the layer.

The relative coordinates are presented in next table (are valid for all 6 TRT part of TRT A). The thickness of active part is 136 mm. The coordinates in table are taken from the center of active part. There is 0.815-mm wide radiator material on both sides of active part.

The thickness column shows the thickness of individual layers; the space column shows space between individual layers (these spaces are filled with foil_material)

| Active part | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] | Thickness[mm] | ### Space[mm] |
| --- | --- | --- | --- | --- | --- | --- |
| Layer 1 | 643.70 | 1004.00 | -67.185 | -59.815 | 7.37 | 0.63 |
| Layer 2 | 643.70 | 1004.00 | -59.185 | -51.815 | 7.37 | 0.63 |
| Layer 3 | 643.70 | 1004.00 | -51.185 | -43.815 | 7.37 | 0.63 |
| Layer 4 | 643.70 | 1004.00 | -43.185 | -35.815 | 7.37 | 2.63 |
| Layer 5 | 643.70 | 1004.00 | -33.185 | -25.815 | 7.37 | 0.63 |
| Layer 6 | 643.70 | 1004.00 | -25.185 | -17.815 | 7.37 | 0.63 |
| Layer 7 | 643.70 | 1004.00 | -17.185 | -9.815 | 7.37 | 0.63 |
| Layer 8 | 643.70 | 1004.00 | -9.185 | -1.815 | 7.37 | 3.63 |
| Layer 9 | 643.70 | 1004.00 | 1.815 | 9.185 | 7.37 | 0.63 |
| Layer 10 | 643.70 | 1004.00 | 9.815 | 17.185 | 7.37 | 0.63 |
| Layer 11 | 643.70 | 1004.00 | 17.815 | 25.185 | 7.37 | 0.63 |
| Layer 12 | 643.70 | 1004.00 | 25.815 | 33.185 | 7.37 | 2.63 |
| Layer 13 | 643.70 | 1004.00 | 35.815 | 43.185 | 7.37 | 0.63 |
| Layer 14 | 643.70 | 1004.00 | 43.815 | 51.185 | 7.37 | 0.63 |
| Layer 15 | 643.70 | 1004.00 | 51.815 | 59.185 | 7.37 | 0.63 |
| Layer 16 | 643.70 | 1004.00 | 59.815 | 67.185 | 7.37 |  |

The layers are filled by:

- Radiator stack

- Straws - tube with radius 2.085 that is positioned in the middle of the layer

- Radiator stack

It means, that there are situated 786 straw in each layer, and the rest of layer is filled by radiator stack

In this table, there is written the position of straw in layer 1. The Z-coordinates are relative. They are taken from the center of active part.

| Layer 1 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Radiator stack | 643.70 | 988.30 | -67.185 | -65.585 |
| Straw | 643.70 | 1004.00 | -65.585 | -61.415 |
| Radiator stack | 643.70 | 988.30 | -61.415 | -59.815 |

Straw consists of:

- Straw wall – tube with inner radius = 2 mm and outer radius = 2.085 mm

- Inside straw – tube with outer radius = 2 mm

Inside straw is filled by:

- Wire – tube with outer radius 0.015 mm

- Dead region – tube (inner radius = 0.015 mm, outer radius = 2 mm, length = 3 mm)

- Gas – tube ( inner radius = 0.015 mm, outer radius = 2 mm, length = 348.3 mm)

- Dead region (inner radius = 0.015 mm, outer radius = 2 mm, length = 3 mm)

The Z-coordinates are relative, that are taken from the middle of straw, the R-coordinates are real

| Straw | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Wire | 643.70 | 1004.00 | 0.00 | 0.015 |
| Dead region | 643.70 | 649.70 | 0.015 | 2.00 |
| Gas | 649.70 | 998.00 | 0.015 | 2.00 |
| Dead region | 998.00 | 1004.00 | 0.015 | 2.00 |
| Straw wall | 643.70 | 1004.00 | 2.00 | 2.082 |

- TRT B

TRT B consists of:

- CO2 Manifold

- Detector part

- CO2 Manifold

|  | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| CO2 Manifold | 620.00 | 1076.50 | 1719.75 | 1728.75 |
| Detector part | 620.00 | 1076.50 | 1728.75 | 2707.50 |
| CO2 Manifold | 620.00 | 1076.50 | 2707.50 | 2716.50 |

The detector part is divided into:

- 8 TRT parts

- 7 Heat exchanger

The coordinates of TRT parts and heat exchangers are written in next table

|  | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| TRT 1 | 620.00 | 1076.50 | 1728.75 | 1842.75 |
| Heat exchanger | 620.00 | 1027.70 | 1842.75 | 1852.25 |
| TRT 2 | 620.00 | 1076.50 | 1852.25 | 1966.25 |
| Heat exchanger | 620.00 | 1027.70 | 1966.25 | 1975.75 |
| TRT 3 | 620.00 | 1076.50 | 1975.75 | 2089.75 |
| Heat exchanger | 620.00 | 1027.70 | 2089.75 | 2099.25 |
| TRT 4 | 620.00 | 1076.50 | 2099.25 | 2213.25 |
| Heat exchanger | 620.00 | 1027.70 | 2213.25 | 2222.75 |
| TRT 5 | 620.00 | 1076.50 | 2222.75 | 2336.75 |
| Heat exchanger | 620.00 | 1027.70 | 2336.75 | 2346.25 |
| TRT 6 | 620.00 | 1076.50 | 2346.25 | 2460.25 |
| Heat exchanger | 620.00 | 1027.70 | 2460.25 | 2469.75 |
| TRT 7 | 620.00 | 1076.50 | 2469.75 | 2583.75 |
| Heat exchanger | 620.00 | 1027.70 | 2583.75 | 2593.25 |
| TRT 8 | 620.00 | 1076.50 | 2593.25 | 2707.25 |

Each TRT part is composed from:

- Inner support

- Active part

- Outer support

- Electronics

The coordinates of TRT 1 are written in next table

| TRT 1 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Inner support | 620.00 | 643.70 | 1728.75 | 1842.75 |
| Active part | 643.70 | 1004.00 | 1728.75 | 1842.75 |
| Outer support | 1004.00 | 1027.70 | 1728.75 | 1842.75 |
| Electronics | 1027.70 | 1076.50 | 1728.75 | 1842.75 |

The active part is segmented into 8 layers. Each layer consists 768straws that are situated regularly in the layer.

The relative coordinates are presented in next table (are valid for all 8 TRT part of TRT B). The thickness of active part is 114 mm.  The coordinates in table are taken from the center of active part.

The thickness column shows the thickness of individual layers; the space column shows space between individual layers (these spaces are filled with foil_material).

| Active part | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] | Thickness[mm] | Space[mm] |
| --- | --- | --- | --- | --- | --- | --- |
| Layer 1 | 643.70 | 1004.00 | -57.00 | -45.415 | 11.585 | 1.83 |
| Layer 2 | 643.70 | 1004.00 | -43.585 | -30.415 | 13.17 | 1.83 |
| Layer 3 | 643.70 | 1004.00 | -28.585 | -15.415 | 13.17 | 1.83 |
| Layer 4 | 643.70 | 1004.00 | -13.585 | -0.415 | 13.17 | 0.83 |
| Layer 5 | 643.70 | 1004.00 | 0.415 | 13.585 | 13.17 | 1.83 |
| Layer 6 | 643.70 | 1004.00 | 15.415 | 28.585 | 13.17 | 1.83 |
| Layer 7 | 643.70 | 1004.00 | 30.415 | 43.585 | 13.17 | 1.83 |
| Layer 8 | 643.70 | 1004.00 | 45.415 | 57.00 | 11.585 |  |

The layers are filled by

- Radiator stack

- Straw - tube with radius 2.085 that is positioned in the middle of the layer

- Radiator stack

It means, that there are situated 786 straw in each layer, and the rest of layer is filled by radiator stack

There are two kinds of layers in active part:

- Layer 1,8 – 11.585 mm wide

- Layer 2, 3, 4, 5, 6, 7 – 13.17 mm wide

The relative coordinates are written in following tables (Z=0 is taken in middle of the layer, R-coordinates are real)

| Layer 1,8 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] | Thickness[mm] |
| --- | --- | --- | --- | --- | --- |
| Radiator stack | 643.70 | 988.30 | -5.7925 | -2.8775 | 2.915 |
| Straw | 643.70 | 1004.00 | -2.8775 | 1.2925 | 4.17 |
| Radiator stack | 643.70 | 988.30 | 1.2925 | 5.7925 | 4.50 |

| Layer 2, 3, 4, 5, 6, 7 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] | Thickness[mm] |
| --- | --- | --- | --- | --- | --- |
| Radiator stack | 643.70 | 988.30 | -6.585 | -2.085 | 4.50 |
| Straw | 643.70 | 1004.00 | -2.085 | 2.085 | 4.17 |
| Radiator stack | 643.70 | 988.30 | 2.085 | 6.585 | 4.50 |

The next tables show relative coordinates for layer 1 and layer 2 taken from the center of active part.

| Layer 1 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] | Thickness[mm] |
| --- | --- | --- | --- | --- | --- |
| Radiator stack | 643.70 | 988.30 | -57.00 | -54.085 | 2.915 |
| Straw | 643.70 | 1004.00 | -54.085 | -49.915 | 4.17 |
| Radiator stack | 643.70 | 988.30 | -49.915 | -45.415 | 4.50 |

| Layer 2 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] | Thickness[mm] |
| --- | --- | --- | --- | --- | --- |
| Radiator stack | 643.70 | 988.30 | -43.585 | -39.085 | 4.50 |
| Straw | 643.70 | 1004.00 | -39.085 | -34.915 | 4.17 |
| Radiator stack | 643.70 | 988.30 | -34.915 | -30.415 | 4.50 |

Straw consists of:

- Straw wall – tube with inner radius = 2 mm and outer radius = 2.085 mm

- Inside straw – tube with outer radius = 2 mm

Inside straw is filled by:

- Wire – tube with outer radius 0.015 mm

- Dead region – tube (inner radius = 0.015 mm, outer radius = 2 mm, length = 3 mm)

- Gas – tube ( inner radius = 0.015 mm, outer radius = 2 mm, length = 348.3 mm)

- Dead region (inner radius = 0.015 mm, outer radius = 2 mm, length = 3 mm)

The Z-coordinates are relative that are taken from the middle of straw, the R-coordinates are real

| Straw | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Wire | 643.70 | 1004.00 | 0.00 | 0.015 |
| Dead region | 643.70 | 649.70 | 0.015 | 2.00 |
| Gas | 649.70 | 998.00 | 0.015 | 2.00 |
| Dead region | 998.00 | 1004.00 | 0.015 | 2.00 |
| Straw wall | 643.70 | 1004.00 | 2.00 | 2.028 |

- TRT C

TRT C consists of:

- CO2 Manifold

- Detector part

- CO2 Manifold

|  | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| CO2 Manifold | 460.00 | 1034.00 | 2815.7 | 2824.75 |
| Detector part | 460.00 | 1034.00 | 2824.75 | 3397.25 |
| CO2 Manifold | 460.00 | 1034.00 | 3397.25 | 3406.25 |

The detector part is divided into:

- 4 TRT part

- 3 Heat exchanger

The coordinates of TRT parts and heat exchangers are in next table

| Detector part | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| TRT 1 | 460.00 | 1034.00 | 2824.74 | 2960.74 |
| Heat exchanger | 460.00 | 994.60 | 2960.74 | 2970.25 |
| TRT 2 | 460.00 | 1034.00 | 2970.25 | 3106.25 |
| Heat exchanger | 460.00 | 994.60 | 3106.25 | 3115.75 |
| TRT 3 | 460.00 | 1034.00 | 3115.75 | 3251.75 |
| Heat exchanger | 460.00 | 994.60 | 3251.75 | 3261.25 |
| TRT 4 | 460.00 | 1034.00 | 3261.25 | 3397.25 |

Each TRT part is composed from:

- Inner support

- Active part

- Outer support

- Electronics

The coordinates or TRT 1 are written in next table

| TRT 1 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Inner support | 460.00 | 483.60 | 2824.74 | 2960.74 |
| Active part | 483.60 | 971.00 | 2824.74 | 2960.74 |
| Outer support | 971.00 | 994.60 | 2824.74 | 2960.74 |
| Electronics | 994.60 | 1034.00 | 2824.74 | 2960.74 |

The active part is segmented into 16 layers. Each layer consists 576 straws that are situated regularly in the layer

The relative coordinates are presented in next table (are valid for all 4 TRT part of TRT C). The thickness of active part is 136 mm. The coordinates in table are taken from the center of active part. There is 0.815-mm radiator material on both sides of active part.

The thickness column shows the thickness of individual layers, the space column shows space between individual layers (these spaces are filled with radiator material)

| Active part | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] | Thickness[mm] | Space[mm] |
| --- | --- | --- | --- | --- | --- | --- |
| Layer 1 | 483.60 | 971.00 | -67.185 | -59.815 | 7.37 | 0.63 |
| Layer 2 | 483.60 | 971.00 | -59.185 | -51.815 | 7.37 | 0.63 |
| Layer 3 | 483.60 | 971.00 | -51.185 | -43.815 | 7.37 | 0.63 |
| Layer 4 | 483.60 | 971.00 | -43.185 | -35.815 | 7.37 | 2.63 |
| Layer 5 | 483.60 | 971.00 | -33.185 | -25.815 | 7.37 | 0.63 |
| Layer 6 | 483.60 | 971.00 | -25.185 | -17.815 | 7.37 | 0.63 |
| Layer 7 | 483.60 | 971.00 | -17.185 | -9.815 | 7.37 | 0.63 |
| Layer 8 | 483.60 | 971.00 | -9.185 | -1.815 | 7.37 | 3.63 |
| Layer 9 | 483.60 | 971.00 | 1.815 | 9.185 | 7.37 | 0.63 |
| Layer 10 | 483.60 | 971.00 | 9.815 | 17.185 | 7.37 | 0.63 |
| Layer 11 | 483.60 | 971.00 | 17.815 | 25.185 | 7.37 | 0.63 |
| Layer 12 | 483.60 | 971.00 | 25.815 | 33.185 | 7.37 | 2.63 |
| Layer 13 | 483.60 | 971.00 | 35.815 | 43.185 | 7.37 | 0.63 |
| Layer 14 | 483.60 | 971.00 | 43.815 | 51.185 | 7.37 | 0.63 |
| Layer 15 | 483.60 | 971.00 | 51.815 | 59.185 | 7.37 | 0.63 |
| Layer 16 | 483.60 | 971.00 | 59.815 | 67.185 | 7.37 |  |

The layers are filled by:

- Radiator stack

- Straw – tube with radius 2.085 mm that is positioned in the middle of the layer

- Radiator stack

It means, that there are situated 786 straw in each layer, and the rest of layer is filled by radiator stack

| Layer 1 | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Radiator stack | 483.60 | 955.30 | -67.185 | -65.585 |
| Straw | 483.60 | 971.00 | -65.585 | -61.415 |
| Radiator stack | 483.60 | 955.30 | -61.454 | -59.815 |

Straw consists of:

- Straw wall – tube with inner radius 2 mm and outer radius 2.085 mm

- Inside straw – tube with outer radius 2 mm

Inside straw is filled by:

- Wire – tube with outer radius 0.015 mm

- Dead region – tube (inner radius = 0.015 mm, outer radius = 2 mm, length = 3 mm)

- Gas – tube (inner radius=0.015 mm, outer radius =2.0 mm, length = 481.4 mm)

- Dead region – tube (inner radius = 0.015 mm, outer radius = 2 mm, length = 3 mm)

The Z-coordinates are relative that are taken from the middle of straw, the R-coordinates are real

| Straw | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Wire | 483.60 | 971.00 | 0.00 | 0.015 |
| Dead region | 483.60 | 486.60 | 0.015 | 2.00 |
| Gas | 486.60 | 968.00 | 0.015 | 2.00 |
| Dead region | 968.00 | 971.00 | 0.015 | 2.00 |
| Straw wall | 483.60 | 971.00 | 2.00 | 2.028 |

# Material composition of TRT

| Material | Elements | Parameters | #### Comments | #### Part of TRT detector |
| --- | --- | --- | --- | --- |
| Straw_wall | C | 0.74 | Weight percentage | Straw wall |
|  | H | 0.022 |  |  |
|  | O | 0.171 |  |  |
|  | N | 0.058 |  |  |
|  | Al | 0.009 |  |  |
| Inner support | CO2 | 1 | Weight percentage | Inner support |
| Outer support | CO2 | 1 | Weight percentage | Outer support |
| Radiator in barrel | CH2 | 1 | Weight percentage | Radiator in barrel |
| Inactive layer in barrel | C | 1 | Weight percentage | Inactive layer in barrel |
| Wire | W | Tube – 15 m |  | Wire |
|  | Au | Tube 15-15.5 m |  | Wire |
| ULTEM | C | 36 | Compound multiplicity |  |
|  | H | 24 |  |  |
|  | O | 6 |  |  |
|  | N | 2 |  |  |
| Dead_region | XE | 1x0.7 | 70% Xe + 20% CF4 + 10% CO2 | Dead region filling |
|  | C | 1x0.2 |  |  |
|  | F | 4x0.2 |  |  |
|  | C | 1x0.1 |  |  |
|  | O | 2x0.1 |  |  |
| Gas | Xe | 1x0.7 | 70% Xe + 20% CF4 + 10% CO2 | Gas compound |
|  | C | 1x0.2 |  |  |
|  | F | 4x0.2 |  |  |
|  | C | 1x0.1 |  |  |
|  | O | 2x0.1 |  |  |
| Foil_radiator | C | 1x0.97 |  | Radiator stack, radiator in EC |
|  | H | 2x0.97 |  |  |
|  | C | 1x0.03 |  |  |
|  | O | 2x0.03 |  |  |
| Inactive layer 21, 45 | C | 1 | Weight percentage |  |

Other parts:

|  | Rmin[mm] | Rmax[mm] | Zmin[mm] | Zmax[mm] |
| --- | --- | --- | --- | --- |
| Squirrel cage 1 | 610.00 | 1076.50 | 2716.50 | 2726.50 |
| Squirrel cage 2 | 800.00 | 1076.50 | 2726.50 | 2745.50 |
| Cable tray, support ring | 840.00 | 1034.00 | 2801.70 | 2815.70 |

Electronics

Geometry is unknown, approximation of dangerous material is written at the end of this file.

Cables and connectors

Barrel part

High Voltage cables

| HV1 cables | Module 1 | Module 2 | Module 3 |
| --- | --- | --- | --- |
| Outer diameter | 1.15 | 1.15 | 1.15 |
| Area[mm2] | 1.04 | 1.04 | 1.04 |
| Number per module | 14 | 12 | 10 |
| Modularity | 16 | 16 | 16 |
| Z [mm] | 850 | 850 | 850 |
| R [mm] | 600 | 750 | 900 |
| Total area [cm2] | 4.70 | 4.00 | 3.30 |

| PPF1->PPF2 | HV 1 Cables | HV 1 Cables - spare | HV 2 Cables |
| --- | --- | --- | --- |
| Outer diameter | 1.15 | 1.15 | 0.97 |
| Area[mm2] | 1.04 | 1.04 | 0.74 |
| Number per module | 36 | 1 | 2 |
| Modularity | 16 | 16 | 16 |
| Z1[mm] | 850 | 850 | 850 |
| Z2[mm] | 3400 | 3400 | 3400 |
| R1[mm] | 1150 | 1150 | 1150 |
| R2[mm] | 3170 | 3170 | 3170 |
| Total area [cm2] | 12.00 | 0.30 | 0.40 |

Low voltage cables

| LV1 cables | Module 1 | Module 2 | Module 3 |
| --- | --- | --- | --- |
| Outer diameter | 4.50 | 4.50 | 4.50 |
| Area[mm2] | 15.90 | 15.90 | 15.90 |
| Number per module | 1 | 2 | 2 |
| Modularity | 32 | 32 | 32 |
| Z [mm] | 775 | 775 | 775 |
| R [mm] | 600 | 750 | 900 |
| Total area [cm2] | 15.90 | 15.90 | 15.90 |

| LV2 cables | Module 1 | Module 2 | Module 3 |
| --- | --- | --- | --- |
| Outer diameter | 4.80 | 4.80 | 4.80 |
| Area[mm2] | 18.10 | 18.10 | 18.10 |
| Number per module | 1 | 2 | 2 |
| Modularity | 32 | 32 | 32 |
| Z [mm] | 775 | 775 | 775 |
| R [mm] | 600 | 750 | 900 |
| Total area [cm2] | 18.10 | 18.10 | 18.10 |

| PPF1->PPF2 | LV 1 Cables | LV2 Cables |
| --- | --- | --- |
| Outer diameter | 4.50 | 4.80 |
| Area[mm2] | 15.90 | 18.10 |
| Number per module | 5 | 5 |
| Modularity | 32 | 32 |
| Z1[mm] | 775 | 775 |
| Z2[mm] | 3400 | 3400 |
| R1[mm] | 1150 | 1150 |
| R2[mm] | 3170 | 3170 |
| Total area [cm2] | 38.90 | 44.20 |

Signal cables

| Data cables | Module 1 | Module 2 | Module 3 | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Outer diameter[mm] | 0.95 | 0.95 | 0.95 | 0.95 |
| Area[mm2] | 0.71 | 0.71 | 0.71 | 0.71 |
| Number per module | 21 | 34 | 50 | 105 |
| Modularity | 32 | 32 | 32 | 32 |
| Z[mm] | 775 | 775 | 775 | 3170 |
| R[mm] | 600 | 750 | 900 | 1150 |
| Total area[cm2] | 9.50 | 15.40 | 22.70 | 47.60 |

| Timing & parameter control | Module 1 | Module 2 | Module 3 | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Outer diameter[mm] | 0.95 | 0.95 | 0.95 | 0.95 |
| Area[mm2] | 0.71 | 0.71 | 0.71 | 0.71 |
| Number per module | 7 | 14 | 14 | 35 |
| Modularity | 32 | 32 | 32 | 32 |
| Z[mm] | 775 | 775 | 775 | 3170 |
| R[mm] | 600 | 750 | 900 | 1150 |
| Total area[cm2] | 3.17 | 6.35 | 6.35 | 15.87 |

| Temperatures cables | Module 1 | Module 2 | Module 3 | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Outer diameter[mm] | 0.95 | 0.95 | 0.95 | 0.95 |
| Area[mm2] | 0.71 | 0.71 | 0.71 | 0.71 |
| Number per module | 4 | 4 | 4 | 12 |
| Modularity | 32 | 32 | 32 | 32 |
| Z[mm] | 775 | 775 | 775 | 3170 |
| R[mm] | 600 | 750 | 900 | 1150 |
| Total area[cm2] | 1.81 | 1.81 | 1.81 | 5.43 |

| Senses cables | Module 1 | Module 2 | Module 3 | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Outer diameter[mm] | 0.95 | 0.95 | 0.95 | 0.95 |
| Area[mm2] | 0.71 | 0.71 | 0.71 | 0.71 |
| Number per module | 3 | 6 | 6 | 12 |
| Modularity | 32 | 32 | 32 | 32 |
| Z[mm] | 775 | 775 | 775 | 3170 |
| R[mm] | 600 | 750 | 900 | 1150 |
| Total area[cm2] | 1.36 | 2.72 | 2.72 | 6.80 |

Forward part

| HV1 | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.15 | 1.15 | 1.15 |
| Area[mm2] | 1.04 | 1.04 | 1.04 |
| Number of wheel | 6 | 8 | 4 |
| Number per wheel | 6 | 3 | 6 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 1240 | 2250 | 3120 |
| R[mm] | 1080 | 1080 | 900 |
| Total area[cm2] | 23.90 | 15.90 | 15.90 |

| HV1 - spare | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.15 | 1.15 | 1.15 |
| Area[mm2] | 1.04 | 1.04 | 1.04 |
| Number of wheel | 6 | 8 | 4 |
| Number per wheel | 1 | 1 | 2 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 1240 | 2250 | 3120 |
| R[mm] | 1080 | 1080 | 900 |
| Total area[cm2] | 4.00 | 5.30 | 5.30 |

PPF1->PP2

| HV1 | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.15 | 1.15 | 1.15 |
| Area[mm2] | 1.04 | 1.04 | 1.04 |
| Number of wheels | 6 | 8 | 4 |
| Number per wheel | 6 | 3 | 6 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 3400 | 3400 | 3400 |
| Z2[mm] | 3400 | 3400 | 3400 |
| R[mm] | 1095 | 1095 | 1095 |
| R2[mm] | 3170 | 3170 | 3170 |
| Total area[cm2] | 23.90 | 15.90 | 15.90 |

| HV1 - spare | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.15 | 1.15 | 1.15 |
| Area[mm2] | 1.04 | 1.04 | 1.04 |
| Number of wheels | 6 | 8 | 4 |
| Number per wheel | 1 | 1 | 2 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 3400 | 3400 | 3400 |
| Z2[mm] | 3400 | 3400 | 3400 |
| R[mm] | 1095 | 1095 | 1095 |
| R2[mm] | 3170 | 3170 | 3170 |
| Total area[cm2] | 4.00 | 5.30 | 5.30 |

| HV2 - interlock | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 0.97 | 0.97 | 0.97 |
| Area[mm2] | 0.74 | 0.74 | 0.74 |
| Number of wheels | 1 | 1 | 1 |
| Number per wheel | 2 | 2 | 2 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 3400 | 3400 | 3400 |
| Z2[mm] | 3400 | 3400 | 3400 |
| R[mm] | 1095 | 1095 | 1095 |
| R2[mm] | 3170 | 3170 | 3170 |
| Total area[cm2] | 0.95 | 0.95 | 0.95 |

Low voltage cables

| LV1 | Wheel A |
| --- | --- |
| Outer diameter[mm] | 4.50 |
| Area[mm2] | 15.90 |
| Number of wheel | 6 |
| Number per wheel | 1 |
| Modularity | 32 |
| Z[mm] | 1240 |
| R[mm] | 1080 |
| Total area[cm2] | 61.10 |

| LV2 | Wheel A |
| --- | --- |
| Outer diameter[mm] | 4.80 |
| Area[mm2] | 18.10 |
| Number of wheel | 6 |
| Number per wheel | 1 |
| Modularity | 32 |
| Z[mm] | 1240 |
| R[mm] | 1080 |
| Total area[cm2] | 69.50 |

| LV3 | Wheel B | Wheel C |
| --- | --- | --- |
| Outer diameter[mm] | 3.45 | 3.45 |
| Area[mm2] | 9.35 | 9.35 |
| Number of wheel | 8 | 4 |
| Number per wheel | 1 | 1 |
| Modularity | 32 | 32 |
| Z[mm] | 2250 | 3120 |
| R[mm] | 1080 | 1080 |
| Total area[cm2] | 47.90 | 23.90 |

| LV4 | Wheel B | Wheel C |
| --- | --- | --- |
| Outer diameter[mm] | 3.65 | 3.65 |
| Area[mm2] | 10.46 | 10.46 |
| Number of wheel | 8 | 4 |
| Number per wheel | 1 | 1 |
| Modularity | 32 | 32 |
| Z[mm] | 2250 | 3120 |
| R[mm] | 1080 | 1080 |
| Total area[cm2] | 47.90 | 26.80 |

| PPF1->PPF2 | LV1 | LV2 | LV3 | LV4 |
| --- | --- | --- | --- | --- |
| Outer diameter[mm] | 4.50 | 4.80 | 3.45 | 3.65 |
| Area[mm2] | 15.90 | 18.10 | 9.35 | 10.46 |
| Number of wheels | 6 | 6 | 12 | 12 |
| Number per wheel | 1 | 1 | 1 | 1 |
| Modularity | 32 | 32 | 32 | 32 |
| Z[mm] | 3400 | 3400 | 3400 | 3400 |
| Z2[mm] | 3400 | 3400 | 3400 | 3400 |
| R[mm] | 1095 | 1095 | 1095 | 1095 |
| R2[mm] | 3170 | 3170 | 3170 | 3170 |
| Total area[cm2] | 61.10 | 69.50 | 71.80 | 80.40 |

Signal cables

| Data cables | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.00 | 1.00 | 1.00 |
| Area[mm2] | 0.71 | 0.71 | 0.71 |
| Number of wheels | 6 | 8 | 4 |
| Number per wheel | 24 | 12 | 18 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 1240 | 2250 | 3120 |
| R[mm] | 1080 | 1080 | 1080 |
| Total area[cm2] | 65.33 | 43.55 | 32.66 |

| Timing & parameters control | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.00 | 1.00 | 1.00 |
| Area[mm2] | 0.71 | 0.71 | 0.71 |
| Number of wheels | 6 | 8 | 4 |
| Number per wheel | 8 | 4 | 8 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 1240 | 2250 | 3120 |
| R[mm] | 1080 | 1080 | 1080 |
| Total area[cm2] | 21.78 | 14.52 | 14.52 |

| Temperatures cables | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.00 | 1.00 | 1.00 |
| Area[mm2] | 0.71 | 0.71 | 0.71 |
| Number of wheels | 6 | 8 | 4 |
| Number per wheel | 4 | 3 | 4 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 1240 | 2250 | 3120 |
| R[mm] | 1080 | 1080 | 1080 |
| Total area[cm2] | 10.89 | 10.89 | 7.26 |

| Senses cables | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.00 | 1.00 | 1.00 |
| Area[mm2] | 0.71 | 0.71 | 0.71 |
| Number of wheels | 6 | 8 | 4 |
| Number per wheel | 3 | 3 | 3 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 1240 | 2250 | 3120 |
| R[mm] | 1080 | 1080 | 1080 |
| Total area[cm2] | 8.17 | 10.89 | 5.44 |

| PPF1->PP2 | Wheel A | Wheel B | Wheel C |
| --- | --- | --- | --- |
| Outer diameter[mm] | 1.00 | 1.00 | 1.00 |
| Area[mm2] | 0.71 | 0.71 | 0.71 |
| Number of wheels | 6 | 8 | 4 |
| Number per wheel | 39 | 22 | 33 |
| Modularity | 32 | 32 | 32 |
| Z[mm] | 775 | 775 | 775 |
| Z2[mm] | 3400 | 3400 | 3400 |
| R[mm] | 1150 | 1150 | 1150 |
| R2[mm] | 3170 | 3170 | 3170 |
| Total area[cm2] | 106.15 | 79.84 | 59.88 |

Fluids

Barrel part

| Ionisation Gas | Module 1 | Module 2 | Module 3 | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Inner diameter[mm] | 5.50 | 5.50 | 5.50 | 5.50 |
| Outer diameter[mm] | 6.00 | 6.00 | 6.00 | 6.00 |
| Area[mm2] | 28.30 | 28.30 | 28.30 | 28.30 |
| Number per module | 1 | 2 | 2 | 5 |
| Modularity | 16 | 16 | 16 | 16 |
| Z[mm] | 775 | 775 | 775 | 775 |
| R[mm] | 600 | 750 | 900 | 1150 |
| Total area[cm2] | 5.88 | 11.76 | 11.76 | 29.4 |

| Straw ventilation | Module 1 | Module 2 | Module 3 | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Inner diameter[mm] | 5.50 | 5.50 | 5.50 | 5.50 |
| Outer diameter[mm] | 6.00 | 6.00 | 6.00 | 6.00 |
| Area[mm2] | 28.30 | 28.30 | 28.30 | 28.30 |
| Number per module | 2 | 2 | 2 | 2 |
| Modularity | 16 | 16 | 16 | 16 |
| Z[mm] | 775 | 775 | 775 | 775 |
| R[mm] | 600 | 750 | 900 | 1150 |
| Total area[cm2] | 11.76 | 11.76 | 11.76 | 11.76 |

| Electronics cooling - retint | Module 1,2,3 | PPF1->PPF2 |
| --- | --- | --- |
| Inner diameter[mm] | 4.50 | 5.50 |
| Outer diameter[mm] | 5.00 | 6.00 |
| Area[mm2] | 19.60 | 28.30 |
| Number per module | 3 | 3 |
| Modularity | 16 | 16 |
| Z[mm] | 775 | 775 |
| R[mm] | 600 | 1150 |
| Total area[cm2] | 12.25 | 17.64 |

| Electronics cooling - supply | Module 1,2,3 | PPF1->PPF2 |
| --- | --- | --- |
| Inner diameter[mm] | 6.50 | 7.50 |
| Outer diameter[mm] | 7.00 | 8.00 |
| Area[mm2] | 38.50 | 50.30 |
| Number per module | 1 | 1 |
| Modularity | 16 | 16 |
| Z[mm] | 775 | 775 |
| R[mm] | 600 | 1150 |
| Total area[cm2] | 8.00 | 10.46 |

Forward region

| Ionisation Gas | Wheel A | Wheel B | Wheel C | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Inner diameter[mm] | 5.50 | 5.50 | 5.50 | 5.50 |
| Outer diameter[mm] | 6.00 | 6.00 | 6.00 | 6.00 |
| Area[mm2] | 28.30 | 28.30 | 28.30 | 28.30 |
| Number per wheel | 4 | 4 | 4 | 4 |
| Number of fed wheels | 6 | 8 | 4 | 18 |
| Z[mm] | 1240 | 2250 | 3120 | 3400 |
| R[mm] | 1095 | 1095 | 1095 | 1095 |
| Total area[cm2] | 8.82 | 11.76 | 5.88 | 26.46 |

| Straw ventilation - supply | Wheel A | Wheel B | Wheel C | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Inner diameter[mm] | 22.00 | 22.00 | 22.00 | 22.00 |
| Outer diameter[mm] | 24.00 | 24.00 | 24.00 | 24.00 |
| Area[mm2] | 452.00 | 452.00 | 452.00 | 452.00 |
| Number per module | 1 | 1 | 1 | 3 |
| Modularity | 0 | 0 | 0 | 0 |
| Z[mm] | 1712 | 2705 | 3395 | 3400 |
| R[mm] | 1080 | 1080 | 1080 | 1095 |
| Total area[cm2] | 4.52 | 4.52 | 4.52 | 13.57 |

| Straw ventilation - return | Wheel A | Wheel A | Wheel B | Wheel B | Wheel C | Wheel C |
| --- | --- | --- | --- | --- | --- | --- |
| Inner diameter[mm] | 5.500 | 24.00 | 5.50 | 24.00 | 5.50 | 24.00 |
| Outer diameter[mm] | 6.00 | 26.00 | 6.00 | 26.00 | 6.00 | 26.00 |
| Area[mm2] | 28.30 | 531.00 | 28.30 | 531.00 | 28.30 | 531.00 |
| Number per module | 2 | 1 | 2 | 1 | 2 | 1 |
| Modularity | 0 | 0 | 0 | 0 | 0 | 0 |
| Z[mm] | 815 | 815 | 1727 | 1727 | 2807 | 2807 |
| R[mm] | 1080 | 1080 | 1080 | 1080 | 1080 | 1080 |
| Total area[cm2] | 5.31 | 5.31 | 5.31 | 5.31 | 5.31 | 5.31 |

| Straw ventilation - return | PPF1->PPF2 | PPF1->PPF2 |
| --- | --- | --- |
| Inner diameter[mm] | 5.50 | 24.00 |
| Outer diameter[mm] | 6.00 | 26.00 |
| Area[mm2] | 28.30 | 531.00 |
| Number per module | 6 | 3 |
| Modularity | 0 | 0 |
| Z[mm] | 3400 | 3400 |
| R[mm] | 1095 | 1095 |
| Total area[cm2] | 2.21 | 15.93 |

| Straw Cooling | Wheel A | Wheel B | Wheel C | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Inner diameter[mm] | 5.50 | 5.50 | 5.50 | 5.50 |
| Outer diameter[mm] | 6.00 | 6.00 | 6.00 | 6.00 |
| Area[mm2] | 28.30 | 28.30 | 28.30 | 28.30 |
| Number per module | 2 | 2 | 2 | 2 |
| Modularity | 5 | 7 | 3 | 15 |
| Z[mm] | 1240 | 2250 | 3120 | 3400 |
| R[mm] | 1080 | 1080 | 1080 | 1095 |
| Total area[cm2] | 3.68 | 5.15 | 2.21 | 11.03 |

| Electronics cooling - supply | Wheel A | Wheel B | Wheel C | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Inner diameter[mm] | 7.50 | 7.50 | 7.50 | 7.50 |
| Outer diameter[mm] | 8.00 | 8.00 | 8.00 | 8.00 |
| Area[mm2] | 50.27 | 50.27 | 50.27 | 50.27 |
| Number per module | 2 | 2 | 2 | 2 |
| Modularity | 6 | 8 | 4 | 18 |
| Z[mm] | 1240 | 2250 | 3120 | 3400 |
| R[mm] | 1080 | 1080 | 1080 | 1095 |
| Total area[cm2] | 7.84 | 10.46 | 5.23 | 23.52 |

| Electronics cooling - return | Wheel A | Wheel B | Wheel C | PPF1->PPF2 |
| --- | --- | --- | --- | --- |
| Inner diameter[mm] | 7.50 | 7.50 | 7.50 | 7.50 |
| Outer diameter[mm] | 8.00 | 8.00 | 8.00 | 8.00 |
| Area[mm2] | 50.27 | 50.27 | 50.27 | 50.27 |
| Number per module | 2 | 2 | 2 | 2 |
| Modularity | 6 | 8 | 4 | 18 |
| Z[mm] | 1240 | 2250 | 3120 | 3400 |
| R[mm] | 1080 | 1080 | 1080 | 1095 |
| Total area[cm2] | 7.84 | 10.46 | 5.23 | 23.52 |

Material composition and dimensions of cables

| HV1 | Number | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- | --- |
| Conductor | 1 | 0 | 0.16 | Cu |
| Dielectric | 1 | 0.16 | 0.52 | HFI150 |
| Drains | 3 | 0 | 0.1 | Cu |
| Shielding | 1 | 0.77 | 0.85 | Al |
| Shell | 1 | 0.85 | 1.15 | HFI150 |

| LV1 | Number | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- | --- |
| Conductor | 3 | 0 | 1.13 | Cu |
| Dielectric | 3 | 1.13 | 1.8 | Polyimide |
| Shielding | 1 | 4.26 | 4.5 | Cu |

| LV2 | Number | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- | --- |
| Conductor | 2 | 0 | 1.38 | Cu |
| Dielectric | 2 | 1.38 | 2.1 | Polyimide |
| Shielding | 1 | 4.56 | 4.8 | Cu |

| LV3 | Number | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- | --- |
| Conductor | 3 | 0 | 0.71 | Cu |
| Dielectric | 3 | 0.71 | 1.3 | Polyimide |
| Shielding | 1 | 3.16 | 3.4 | Cu |

| LV4 | Number | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- | --- |
| Conductor | 2 | 0 | 0.87 | Cu |
| Dielectric | 2 | 0.87 | 3.1 | Polyimide |
| Shielding | 1 | 3.36 | 3.6 | Cu |

| Signal cables type C | Number | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- | --- |
| Conductor | 2 |  | 0.16 | Al |
| Dielectric | 2 | 0.22 | 0.47 | HFI150 |
| Drains | 1 |  | 0.18 | Al |
| Shielding inner | 1 | 0.98 | 1.02 | Al |
| Shielding outer | 1 | 1.02 | 1.06 | Polyester |

| Signal cable OS1 | Number | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- | --- |
| Conductor | 2 |  | 0.13 | Cu |
| Dielectric | 2 | 0.13 | 0.39 | HFI150 |
| Drains | 4 | 0 | 0.0635 | Cu |
| Shielding | 1 | 0.86 | 0.95 | Al |

Connectors

Types of connectors

| High voltage | Contacts |
| --- | --- |
| Special Lemo | 3 |
| Lemo/Redel | 51 |
| Low voltage |  |
| Radiall | 34 |
| Datas |  |
| Robinson | 60 |
| Robinson | 51 |
| Robinson | 34 |
| Connectors of fluid |  |
| Standard straight | Tube OD 6 |
|  | Tube OD 8 |
| Standard elbow | Tube OD 6 |
|  | Tube OD 8 |

Material composition and dimensions of connectors

| Connector HV Lemo 51 pines | ID[mm] | OD[mm] | Length[mm] | Height[mm] | Material |
| --- | --- | --- | --- | --- | --- |
| Contacts | 0.00 | 1.15 |  |  | Bronze |
| Corps |  |  | 43.00 | 18.60 | Peek |
| Shielding |  |  | 43.00 | 0.50 | Al |
| Connector (total) |  |  | 54.00 | 20.00 |  |

| Connector LV RADIALL 34 pines | ID[mm] | OD[mm] | Length[mm] | Height[mm] | Material |
| --- | --- | --- | --- | --- | --- |
| Contacts | 0.00 | 1.50 |  |  | Cu |
| Corps |  |  | 61.00 | 20.00 | Resine |
| Shielding |  |  | 63.00 | 1.00 | Al |
| Connector (total) |  |  | 70.00 | 22.00 |  |

| Connector Datas-Robinson 60 pines | ID[mm] | OD[mm] | Length[mm] | Height[mm] | Material |
| --- | --- | --- | --- | --- | --- |
| Contacts |  |  | 0.60 | 0.40 | Cu |
| Corps |  |  | 44.00 | 7.00 | Plastic |
| Circuit |  |  | 54.00 | 1.60 | Glass fibre |
| Shielding |  |  | 54.00 | 0.60 | Al |
| Connector (total) |  |  | 59.00 | 9.00 |  |

| Connector Senses-Robinson 50 pines | ID[mm] | OD[mm] | Length[mm] | Height[mm] | Material |
| --- | --- | --- | --- | --- | --- |
| Contacts |  |  | 0.60 | 0.40 | Cu |
| Corps |  |  | 37.00 | 7.00 | Plastic |
| Circuit |  |  | 54.00 | 1.60 | Glass fibre |
| Shielding |  |  | 54.00 | 0.60 | Al |
| Connector (total) |  |  | 59.00 | 9.0 |  |

| Connector temperatures-Robinson 34 pines | ID[mm] | OD[mm] | Length[mm] | Height[mm] | Material |
| --- | --- | --- | --- | --- | --- |
| Contacts |  |  | 0.60 | 0.40 | Cu |
| Corps |  |  | 27.00 | 7.00 | Plastic |
| Circuit |  |  | 47.00 | 1.60 | Glass fibre |
| Shielding |  |  | 47.00 | 0.60 | Al |
| Connector (total) |  |  | 47.00 | 9.00 |  |

| Fluid connector - tube OD6 | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- |
| Connection | 6.00 | 14.00 | S steel |

| Fluid connector - tube OD8 | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- |
| Connection | 8.00 | 17.00 | S steel |

| Fluid connector - tube OD6 | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- |
| Connection | 6.00 | 14.00 | S steel |

| Fluid connector - tube OD8 | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- |
| Connection | 8.00 | 17.00 | S steel |

| C6F14 connector | ID[mm] | OD[mm] | Material |
| --- | --- | --- | --- |
| Connection | 6.00 |  | S steel |

# Dangerous material present in TRT

This information is take from Fred Luehring material file.

##### Total amount of dangerous material at the end of barrel (most of material is electronics and support)

The geometry of electronics is now unknown. The approximate weights (just estimation!!!!!!) of dangerous material of individual parts are written in next tables. Aluminum is present in next tables due to impurities.

| Barrel | Material | Weight (g) |
| --- | --- | --- |
| HV Plate Ground | Cu | 406.34 |
| HV Plate Solder | Pb/Sn | 52.21 |
| Cap socket - upper | Brass | 94.06 |
| Cap socket - lower | Brass | 188.12 |
| Cap Double Pin | Brass | 264.96 |
| Cap Top Wire | Cu | 132.48 |
| Capacitors | BaTiO2 | 788.26 |
| Conductive Epoxy | Silver | 525.44 |
| Fuses | Cu | 101.76 |
| Fuses Box Capacitors | BaTiO2 | 69.12 |
| Spring Clip in Fuse Box | Brass | 57.6 |
| Mounting Pins | Steel | 78.72 |
| Tension Plate | Cu | 790.1 |
| Eyelets | Brass | 1050.88 |
| Taper Pin | Brass | 2028.2 |
| ASDBLR Signal Socket | Brass | 958.46 |
| ASDBLR Ground Socket | Brass | 359.42 |
| ASDBLR Pads | Cu | 835.53 |
| ASDBLR Bond Wires | Au | 13.77 |
| ASDBLR Ground Plane | Cu | 329.63 |
| DMTROC Pads | Cu | 804.51 |
| DMTROC Bond Wires | Au | 8.07 |
| DMTROC Ground Plane | Cu | 392.63 |
| Flex board Cu | Cu | 924.27 |
| Brass Pins for Tension Plate | Brass | 1257.98 |
| Resistors | Cu | 168.9 |
| White Connector Male | Cu | 469.65 |
| Snake Cable Cu | Cu | 53.05 |
| White Connector Female | Cu | 1043.66 |
| Signal Connector Pins | Cu | 157.7 |
| Cooling Tabs | Al | 2995.2 |
| Cooling Plate | Al | 9394.88 |
| Periscope Fitting | Al | 983.04 |
| Cooling Fluid Manifold | S Steel | 1171.2 |
| Cooling Manifold Female F. | S Steel | 548.51 |
| Cooling Manifold Male F. | S Steel | 210.48 |
| Sense Gas Manifold | S Steel | 945.6 |
| Reducer | S Steel | 104.83 |
| Stainless steel tubes | S Steel | 42.24 |
| Transition Fitting | S Steel | 115.97 |
| Swage Band | S Steel | 6.14 |
| Vent Gas Manifold | S Steel | 449.16 |

The total amount of individual elements from previous table is written in next table.

| Material | Cu | Brass | Solder | Au | Silver | BaTiO2 | Steel | Al |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Weight (g) | 6613.69 | 6259.68 | 52.21 | 21.84 | 525.44 | 857.38 | 3672.8 | 13373 |

The composition of brass:

- Zinc (28-35%)

- Cu (60-70 %)

- Pb, Sn, Fe (less than 5%)

The exact amount depends on type of brass. There is approximate amount of elements in brackets.

The approximate weight of rest electronic barrel part is written in next table.

| Material | ULTEM | G10 | MYLAR | SiO2 | Kapton | Plastic | PEEK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Weight [g] | 5770.92 | 9596.01 | 15118.56 | 470.5 | 2091.79 | 3377.54 | 1126.63 |

| Material | H | C | O | N | Si |
| --- | --- | --- | --- | --- | --- |
| ULTEM | 4.2 | 74.5 | 16.5 | 4.8 | 0 |
| G10 | 1.6 | 25 | 45.3 | unknown |  |
| MYLAR | 4.2 | 62.5 | 33.3 | 0 | 0 |
| SiO2 | 0 | 0 | 53.3 | 0 | 46.7 |
| Kapton | 2.6 | 69.2 | 20.9 | 7.3 | 0 |
| Plastic | unknown |  |  |  |  |
| PEEK | 4.3 | 84.4 | 11.3 | 0 | 0 |

The next information is taken from ATL-INDET-2000-013.  It is approximate of dangerous material, that is present in EC electronics.

|  | Part | Material | Weight[g] | Percentage | Thickness | Part of TRT |
| --- | --- | --- | --- | --- | --- | --- |
| WEB | Solder | Pb | 520.92 | 30 |  | EC |
|  |  | Sn | 780.00 | 70 |  |  |
|  | Main part | Cu | 10000.00 |  |  |  |
| Squirrel cage |  | Al | Unknown |  | 6 mm | EC |
|  |  | Cu | 270.00 | 0.10% |  |  |
|  |  | Fe | 1075.00 | 0.40% |  |  |
|  |  | Cr | 670.00 | 0.25% |  |  |
|  |  | Zn | 670.00 | 0.25% |  |  |
| Chip boards |  | Cu | 10000.00 |  | 35 m | EC |
|  |  | Cu | 5000.00 |  |  | Barrel |
| Solder on boards |  | Pb | 2953.30 | 30 | 135 m | EC |
|  |  | Sn | 4430.00 | 70 |  |  |
|  |  | Pb | 1175.40 | 30 |  |  |
|  |  | Sn | 1760.00 | 70 | 0.1 mm | Barrel |
| Conductive plate |  | Zn | 35.00 | 20% | 2 m | EC |
|  |  | Sn | 45.00 | 25% | 2 m |  |
|  |  | Cu | 95.00 | 55% | 2 m |  |
|  |  | Cr | 155.00 |  |  |  |
| Petals |  | Au | 12.50 | 0.05 m |  | EC |